3GPP TSG-WG SA2 Meeting #168	S2-2503835

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR for TS 23.369: Update Command Procedure

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Clause 6.2.3 is updated to add more details for the AIoT command procedure.

1. Introduction/Discussion

This contribution updates the Command procedure to add more details to clarify how to use the parameters accordingly, and how to inform the End of Command Procedure. In addition, there are several Editor’s Notes in the clauses which are to be addressed by this contribution.

Editor's note:	It is FFS whether the AIoT RAN performs aggregation for Inventory Report during the Command procedure.

For this Editor’s note, further clarification on how to perform command operation if NG-RAN performs aggregation for Inventory Report during the Command Procedure.

When the NG-RAN performs aggregation, inventory reports are consolidated into one or more N2 messages. Whether NG-RAN performs the aggregation is based on RAN’s implementation.

Editor's note:	It is FFS how to handle the scenarios that the Inventory Response received for Command Request sent for many individual devices are in parallel.

For this Editor’s note, when Inventory Response received for Command Request sent for many individual devices are in parallel, the NG-RAN shall be able to distinguish downlink Command Requests for individual devices.

In order to allow NG-RAN to determine which AIoT Device a Downlink Command Request is for the AIoT RAN generates a unique AIoT RAN NGAP ID for each device reported in the inventory reports. These AIoT RAN NGAP IDs are included alongside the corresponding AIoT NAS message in the Downlink Command Request from the AIOTF, allowing the NG-RAN to accurately route the command to the intended AIoT Device.

Editor's note:	Further clarification and enhancement on step 7 to step 11 of procedure for Inventory specified in clause 6.2.2 is FFS.

For this Editor’s note, further clarification and enhancement between Command Procedure and step 7 to step 11 of procedure for Inventory are added.

In step 7, a command Indication is added in Inventory Request, to inform NG-RAN there may be commands following by inventory procedure.

In step 10, AIoT NGAP ID is added in addition to the Correlation ID, Reader ID and the AIOT NAS message(s).

Editor's note:	It is FFS whether a subscribed service is required before triggering Nnef_AIoT_Command Notify.

For this Editor’s note, an implicit subscribed service can be considered to trigger this Nnef_AIoT_Command Notify.

Apart from the Editor’s note, this contribution also clarifies how to complete the Command Procedure using an end message. When AIOTF/ NG-RAN determines to end the Command procedure when, e.g., all the target AIoT Devices have had commands sent to them, the AIOTF can end Command Procedure and release the context in NG-RAN and itself.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

6.2.3	Procedure for Command

Editor’s Note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor’s Note:	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.X.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], (AIoT data)) to NEF.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

The Target AIoT device information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

Editor's note:	It is FFS how to define the filtering information.

2.	The NEF selects AIOTF(s) taking into account the target area information for AIoT service operation and Target AIoT device information in the step 1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT Command request with an appropriate cause code. The AIOTF performs AIoT RAN and optionally AIoT RAN Reader selection as specified in clause 5.3. If no AIoT RAN or AIoT RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

Editor’s Note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject result for the AIoT Command service operation request) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject result for the AIoT Read/Write/PermanentDisable service operation request) to the AF.

7.	Step 7 to step 12 of procedure for Inventory specified in clause 6.2.2 are performed, with the following differences in the inventory procedure:

-	In step 7, a Command Indication is added in the Inventory Request, to inform NG-RAN there may be commands after the inventory procedure.

-	In step 10, NG-RAN includes an AIoT RAN NGAP ID for each AIoT Device in addition to the Correlation ID, Reader ID and the AIOT NAS message from each AIoT Device.

-	In step 11, the AIoT validates the results, and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information.

8.	AIOTF sends Command Request message (Correlation ID, AIoT RAN NGAP ID, NAS Command Request) to the AIoT RAN. The Correlation ID is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request. The AIoT RAN NGAP ID is received the Inventory Response and is used by NG-RAN to determine which AIoT Device the Command Request is targeting.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The AIoT RAN sends the NAS Command Request to the AIoT Device. NG-RAN uses the combination of the Correlation ID and the AIoT RAN NGAP ID in the Command Request to determine which AIoT Device to send the NAS Command Request to.

10.	The AIoT device sends a NAS Command Response to the AIoT RAN.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG’s specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

11.	The AIoT RAN responds with a Command Response message (Correlation ID, AIoT RAN NGAP ID, NAS Command Response) to the AIOTF.

12.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Naiotf_AIoT_Notify message (AIoT Device ID(s), AF ID, AIoT data). The AIOTF determines which AIoT Device a NAS Command Response is from from the AIoT RAN NGAP ID included in Command Response from NG-RAN.

13.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Notify message (AIoT Device ID(s), AF ID, AIoT data).

14.	The AIOTF sends a Service End message to NG-RAN, indicating that the Command service operation associated with the correlation ID is completed, e.g., after it determines all the target AIoT Devices has been send commands.

When the Command service operation associated with the correlation ID has completed, the AIOTF and NG-RAN can release the context associated with the correlation ID.

# * * * * End of changes * * * *