3GPP TSG-WG SA2 Meeting #168	S2-2503836

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR for TS 23.369: Improve AIoT NF Services

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: address the open issues to support Ambient IoT services on the NFs.

1. Introduction/Discussion

1.1	Issues for Number of Service Operation

Editor's note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

Editor’s note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

There are two ways to design the AIoT Command service operations, either 1) Single Command service operation + Command Type service parameter, or 2) Three separate Read, Write or Disable command service operation.

Both ways work technically. However, way 2) shows benefits in specification documentation, it enables a more structural and clear way to show the differences (e.g., Command specific parameters, Information to be used for resource allocation) between the Read, Write or Disable command service operations.

If way 1) continues, then it is suggested to have a more structural and clear documentation manner to show the differences for different command operations.

1.2	Issues on AMF transport service to support BS Reader information reporting

Editor's note:	It is FFS how to support the service between AMF and AIOTF in order to report and update the information provided by the RAN for indirect path.

Editor's note:	AIoT messages is FFS and coordination with RAN3 is needed.

Editor’s note:	Further functions on AMF is FFS.

RAN3 would like to inform SA2 and SA5 about the following RAN3 progress related to Architecture and OAM aspects:

- A-IoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

- Support both the direct and indirect communication options between the gNB and the AIOTF in Topology 1.

- Working assumption: Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP.

- It is FFS whether to define a new A-IoT Area, or use Tracking Area dedicated for A-IoT, to support e.g., the selection of gNBs by the AIOTF.

- AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information.

- AIOTF may also be aware of the other A-IoT RAN information (location of reader) via OAM configuration, FFS on RAN3 signalling for the other A-IoT RAN information...

- In Inventory Request, gNB receives the requested Service Area Information (encoded as Area and/or reader ID list) from AIOTF.

- The gNB always sends the A-IoT device’s location at reader ID granularity to the AIOTF in Inventory Report.

- “Global gNB ID + Reader index" is used to uniquely identify the Reader globally.

LS from RAN3 (R3-250905) states that

Observation 1: Container in NGAP is used to transport any AIOTF information. In case of Indirect connectivity between AIOTF and AIoT RAN via an AMF, the AMF performs transparent relay of the AIOTF information in the NGAP Container between AIOTF and AIoT RAN, thus the AIOTF information transferred in the NGAP Container is transparent to the AMF, i.e., the AMF will not interpret any AIOTF information in the NGAP Container.

In case of Indirect connectivity between AIOTF and AIoT RAN via an AMF, NGAP over the N2 reference point between the AIOT RAN and AMF supports Ambient IoT services including delivery of inventory/command messages. Thus, AIOTF information in the NGAP Container includes at least inventory/command messages, which RAN3 specifies the following Container IE:

-	Inventory Request Transfer, Inventory Response Transfer, Inventory Failure Transfer, Command Request Transfer, Command Response Transfer, Command Failure Transfer

-	Inventory Report Transfer

According to RAN3 LS, AIOTF can obtain the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration. RAN3 signalling for the A-IoT RAN information is FFS. However, even though RAN3 defines the signalling to transfer A-IoT RAN information from AIoT RAN to AIOTF, this A-IoT RAN information will be transported in the NGAP Container, thus to align with RAN3 assumption.

Observation 2: Even though RAN3 defines the signalling to transfer A-IoT RAN information from AIoT RAN to AIOTF, this A-IoT RAN information will be transported in the NGAP Container.

Proposal 1: AMF is enhanced to perform transparent relay of any AIOTF information in the NGAP Container between AIOTF and AIoT RAN. AIOTF information includes Container IE defined for inventory/command messages. Whether to support relaying of A-IoT RAN information needs RAN3 progress. If supported, how to design the transport service between AMF and AIOTF will be determined later.

Currently, the message transfer service for N1N2Message and NonUEN2Message doesn’t include the Container in the message transfer response, instead, the container is included in the message transfer notification.

Proposal 2: Move the Container IE from message delivery response service operation to message notify service operation.

1.3	Issues on command specific parameters

Editor’s note:	The AIoT Data specific for the Read/Write/Disable command is FFS.

Editor’s note:	The Command specific parameters for Read/Write/Disable command is FFS.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also be provided by the AF for the other command operations.

AIoT Devices have reduced functionality compared to other types of UE (e.g., NR, eMTC or NB-IoT). An AIoT Device is more akin to a memory device (for example a flash memory card) that utilize basic memory comparison operation (match a bit sequence (e.g., the MASK) against memory location Y for length Z) to support identification, and then further operations for inventory, reading and writing data. The command parameters for read, write, disable command are different as following:

-	For Read Command, it is used to retrieve information from a specific AIoT device. The AF needs to indicate e.g., the memory location to read, the memory length to read. In this case, the AF use the " the memory length to read" to derive the approximate response data size, this is used for resource allocation at AIOT RAN.

-	For Write Command, it is used to configure information into a specific AIoT device. The AF needs to indicate e.g., the memory location to write, the memory length to write, application data to write. The response from AIoT device is expected as acknowledgement to the write command.

-	For Disable Command, it is used to permanently disable the capability of the AIoT device(s) to transmit RF signals. The AF may indicate some security related parameters, e.g., signature and password for the target AIoT device and network to validate this particular command operation. The response from AIoT device is expected as acknowledgement to the disable command. But this is for SA3 to check whether the security protection (e.g., encryption) is already enough for disable command service.

Proposal: Support of "memory location to read", "memory length to read", "approximate response data size" for Read Command. Support of "memory location to write", "memory length to write", "application data to write" for Write Command.

1.4	Issues on notify service operation

Editor’s note:	It is FFS whether that fail to discovery any AIoT device is normal operation result or failure case.

Failure case indicates that the inventory or command procedure cannot be executed normally due to unexpected error, e.g., resource allocation failure. If the procedure completes normally but there is no matched target AIoT device, this should be a normal operation result.

Editor’s note:	It is FFS how to make AF be aware of the report is finished in case there are multiple notify for the same AIoT operation.

Editor's note:	It is FFS how to make AIOTF be aware of the report is finished in case there are multiple notify for the same operation.

When the AIoT RAN detects the completion of the procedure e.g., that there is no further Ambient IoT Device performing random access, collected AIoT devices has exceeded the expected number of AIoT devices, the allocated resources have used up etc., the AIoT RAN can inform the AIOTF of the completion indication of the procedure. This completion indication can be sent along with the last inventory report or sent using a dedicated message signalling.

Meanwhile, the AIOTF can also detect the completion of the procedure, e.g., all target Ambient IoT Device(s) have been inventoried etc. In this case, the AIOTF can either inform the AIoT RAN of the completion indication or disregard the further reports from the AIoT RAN.

Proposal: Notify service operation is used to carry the completion indication to indicate that the procedure is finished.

1.5	Issues on AMF service operation

Editor’s note:	It is FFS whether the Correlation identifier or AIOTF ID is included in the Namf_AIoT_MessageDelivery.

Editor's note:	It is FFS whether the Correlation identifier is included in the Namf_AIoT_Notify.

TR conclusion 1: When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request.

The Correlation ID is used by AIOTF to correlate the results in one or more reports to a given operation request. This Correlation ID shall be part of the Inventory Report or Inventory Response or Command Response.

Proposal 1: No need to add Correlation ID as input parameters of the Namf_AIoT_ notify.

TR conclusion 2: UL traffic is routed based on an AIoT correlation identifier (i.e. AIOT-RAN identifies the appropriate AMF to which to forward UL messages base on the association between AIoT correction identifier and the AMF).

The Correlation ID is used by AIoT-RAN to correlate the NGAP association with AMF to route the UL traffic back. Meanwhile, inventory request, response, failure is paired class 1 NGAP messages. Same for command request, response and failure. If the NGAP association between AIoT-RAN and AMF is used to transfer NGAP Command messages for multiple devices in parallel, then the AMF and AIoT-RAN need to identify different paired NGAP command procedure messages. A device transaction ID is required for this purpose.

TR conclusion 3: The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

The AIOTF identifier is used by AMF to route the responses related to a request back to the AIOTF that triggers the request.

Proposal 2: add both the Correlation identifier and AIOTF identifier as required input parameter for Namf_AIoT_ Messagedelivery service operations.

Proposal 3: add device transaction identifier as required input parameter for Namf_AIoT_Messagedelivery service operations in case it used to transfer AIoT command messages.

1.5	Others

Introduce the general description for network function Ambient IoT services.

In Message Delivery AMF service operation, the AIOTF needs to indicate to the AMF the message type, so the AMF can construct the correct NGAP message header (Inventory or Command) sent to the AIoT RAN.

For ADM AIoT service, the NEF can query the last known AIOTF reader information in AIoT device profile data to optimize the target AIOTF selection. The NEF can be the consumer of ADM AIoT service.

For NEF AIoT services, this needs to align with the AIOTF AIoT services.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

4.5.7	AMF

The AMF performs the following functions when the AIoT RAN and the AIOTF communicate indirectly via an AMF:

-	Relaying signalling for AIoT service (including e.g., inventory/command messages) between AIoT-RAN and AIOTF transparently.

-	Providing transparent transport for AIoT NAS messages between AIoT Device and AIOTF.

# * * * * Second change * * * *

7	Network Functions Services

7.1	General

In the context of Ambient IoT specification, an NF service is offering a capability to authorised upstream and downstream NF consumers, thus to enable the network to offer the end-to-end Ambient IoT services from system perspective.

The following clauses provide for each involved NF the NF services it exposes through its service-based interfaces.

7.2	AIOTF services

7.2.1	General

The AIOTF supports to expose AIoT services towards the AF or the NEF as described in Table 7.2.1-1.

Table 7.2.1-1: NF services provided by the AIOTF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | NEF, AF |
|  | Command | Request/Response | NEF, AF |
|  | Notify | Subscribe/Notify | NEF, AF |

Editor's note:	It is FFS how to support the transport service between AMF and AIOTF in order to report and update the AIoT RAN information (supported Area, served reader ID list, location of the reader), if concluded to support by RAN3 for indirect connectivity case.

7.2.2	Naiotf_AIoT_Inventory service operation

Service operation name: Naiotf_AIoT_Inventory.

Description: The NF consumer requests an inventory operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the inventory operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID or the filtering information for multiple AIoT Devices.

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

7.2.3	Naiotf_AIoT_Command service operation

Service operation name: Naiotf_AIoT_Command.

Description: The NF consumer requests a command operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the command operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID or the filtering information for multiple AIoT Devices.

4)	Command Type: Read, Write or Permenant Disable.

5)	Command specific parameters:

-	For Read Operation: memory address to read, memory length to read.

-	For Write Operation: memory address to write, memory length to write, AIoT data to write.

NOTE:	AIoT Devices have reduced functionality compared to other types of UE (e.g., NR, eMTC or NB-IoT). An AIoT Device is more akin to a memory device (for example a flash memory card) that utilize basic memory comparison operation (match a bit sequence (e.g., the MASK) against memory location Y for length Z) to support identification, and then further operations for inventory, reading and writing data.

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

-	Approximate AIoT data size to read from the AIoT Device for Read Operation.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

7.2.4	Naiotf_AIoT_Notify service operation

Service operation name: Naiotf_AIoT_Notify

Description: The AIOTF uses this service operation to notify the results or status of the service operation towards the NF consumers. If the NF consumer invokes the Naiotf_AIoT_Inventory, or Naiotf_AIoT_Command service operation, the NF consumer implicitly subscribes to the results of the requested service operation.

Inputs, Required:

Transaction ID, Failure Cause in case of Failure.

Inputs, Optional: List of AIoT Device ID, Information obtained from each target AIoT Device in case of read command, Completion Indication.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.3	AMF services

7.3.1	General

AMF supports to expose AIoT services towards the AIOTF as described in Table 7.3.1-1. The Namf_AIoT AMF service is used when the AIOT RAN and the AIoTF communicate indirectly via an AMF.

Table 7.3.1-1: NF services provided by the AMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_AIoT | MessageDelivery | Request/Response | AIOTF |
|  | Notify | Subscribe/Notify | AIOTF |

7.3.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: The NF consumer requests to send AIoT data towards AIOT RAN or AIoT devices.

Inputs, Required:

1)	AIoT data in NGAP Container (Inventory Request Transfer, Command Request Transfer) to deliver to AIoT RAN.

2)	AIOT RAN ID.

3) 	AIoT NGAP Message Type ("Inventory" or "Command").

4) 	AIOTF Identifier, this is to allow the AMF to route back the requesting AIOTF with the response(s) to the delivered request.

5) 	Corelation Identifier, this is to allow identifying the association between AIoT RAN and AMF to route back the response(s) to the delivered request.

6) Device Transaction Identifier, this is to identify a transaction procedure for an individual device over the association between AIoT RAN and AMF if the AIoT NGAP Message Type is set with "Command".

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

7.3.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: The NF consumer requests to receive AIoT data from AIOT RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the AIoT data from AIOT RAN or AIoT devices.

Inputs, Required:

1)	AIoT data in NGAP Container (Inventory Response Transfer, Inventory Failure Transfer, Command Response Transfer, Command Failure Transfer, Inventory Report Transfer) received from AIOT RAN.

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

7.4	NEF services

7.4.1	General

In addition to those defined in clause 7.2.8 of TS 23.501 [3] and clause 5.2.6 of TS 23.502 [4], table 7.4.1-1 illustrates additional NEF services to support AIoT.

Table 7.4.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

7.4.2	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_AIoT_Inventory

Description: The consumer requests to perform an inventory operation for an AIoT Device or multiple AIoT Devices.

Input, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the inventory operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID or the filtering information for multiple AIoT Devices.

Input, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the command operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID or the filtering information for multiple AIoT Devices.

4)	Command Type: Read, Write or Permenant Disable.

5)	Command specific parameters:

-	For Read Operation: memory address to read, memory length to read.

-	For Write Operation: memory address to write, memory length to write, AIoT data to write.

Input, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

-	Approximate AIoT data size to read from the AIoT Device for Read Operation.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.4	Nnef_AIoT_Notify service operation

Service operation name: Nnef_AIoT_Notify

Description: The consumer receives notification of the status or results of the requested service operation. If the consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Command service operation, the consumer implicitly subscribes to the results of the requested service operation.

Input, Required: Transaction ID, Failure Cause in case of Failure.

Input, Optional: List of AIoT Device ID, Information obtained from each target AIoT Device in case of read command, Completion Indication.

Output, Required: Result indication.

Output, Optional: None.

7.5	ADM services

7.5.1	General

The following table shows the Nadm_DM Service and Service Operations.

Table 7.5.1-1: NF services provided by ADM

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_DM | Query | Request/Response | AIOTF, NEF |
|  | Update | Request/Response | AIOTF |

7.5.2	Nadm_DM_Query service operation

Service operation name: Nadm_DM_Query

Description: NF service consumer may request the AIoT device profile data or the AF authorization data from the ADM.

Inputs, Required: AIoT Device Permanent ID, or AF ID.

Input, Optional: None.

Outputs, Required: the AIoT device profile data or the AF authorization data.

Output, Optional: None.

7.5.3	Nadm_DM_Update service operation

Service operation name: Nadm_DM_Update

Description: NF service consumer may update the AIoT device profile data in the ADM.

Inputs, Required: AIoT Device Permanent ID, updated AIoT device profile data.

Input, Optional: None.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

Output, Optional: None.

# * * * * End of changes * * * *