3GPP TSG-WG SA2 Meeting #168	S2-2503837

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR for TS 23.369: Support of AIoT Device Context Management

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: add support of AIoT device context management

1. Introduction/Discussion

Clause 8.1.1 of TR 23.700-13 concludes the support of AIoT device context management

-	The AIOTF may store and manage the AIoT device related information (also known as device context information) locally that includes e.g. the AIOT device permanent ID, the last known reader information of the AIoT device. The last known reader information can be used to support the AIOTF to select the serving reader to forward the message towards the specific AIoT device(s).

It is proposed to add the feature of AIoT device context management.

2. Text Proposal

It is proposed to capture the following changes vs. TS23.369.

# * * * * First change * * * *

5.X	AIoT Device Context Management

The AIOTF supports management of the AIoT device context information locally. The AIoT device context includes e.g., the AIoT Device Permanent ID, the last known AIoT RAN reader information etc. The last known reader information can be used to support the AIOTF to perform AIoT reader selection as defined in clause 5.3.

When the AIOTF receives the Inventory Report from the AIoT RAN,

-	The AIOTF extracts the Inventory Response AIoT NAS message sent from an AIoT Device and determines the AIoT Device Permanent ID. The AIOTF also obtains the AIoT RAN reader ID that represents the AIoT device’s location at reader ID granularity.

-	If there is no available AIoT device context for this AIoT device,

-	The AIOTF retrieves the AIoT device profile data from ADM and checks the AIoT Device Permanent ID against the AIoT device profile.

-	If validated, the AIOTF generates the AIoT device context to store the AIoT Device Permanent ID, the last known AIoT RAN reader information.

-	Otherwise, the AIOTF checks the AIoT Device Permanent ID against the local AIoT device context. The AIOTF may update the last known AIoT RAN reader information in the AIoT device context.

When the AIOTF selects the AIoT RAN node and/or reader using last known AIoT RAN reader information in the AIoT device context but does not find the target AIoT device through the inventory process under last known AIoT RAN reader, then the AIOTF deletes the available AIoT device context of the target AIoT device.

# * * * * End of changes * * * *