3GPP TSG-WG SA2 Meeting #168	S2-2502044

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR for TS 23.369: AIoT Reader Selection

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: address ENs about AIoT Reader Selection

1. Introduction/Discussion

RAN3 would like to inform SA2 and SA5 about the following RAN3 progress related to Architecture and OAM aspects:

- A-IoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

- Support both the direct and indirect communication options between the gNB and the AIOTF in Topology 1.

- Working assumption: Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP.

- It is FFS whether to define a new A-IoT Area, or use Tracking Area dedicated for A-IoT, to support e.g., the selection of gNBs by the AIOTF.

- AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information.

- AIOTF may also be aware of the other A-IoT RAN information (location of reader) via OAM configuration, FFS on RAN3 signalling for the other A-IoT RAN information...

- In Inventory Request, gNB receives the requested Service Area Information (encoded as Area and/or reader ID list) from AIOTF.

- The gNB always sends the A-IoT device’s location at reader ID granularity to the AIOTF in Inventory Report.

- “Global gNB ID + Reader index" is used to uniquely identify the Reader globally.

LS from RAN3 (R3-250905) states that

Observation: As RAN3 concluded, AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list, location of reader) via OAM mechanism. The Inventory Request includes the requested Service Area Information (encoded as Area and/or reader ID list). The Inventory Report includes the A-IoT device’s location at reader ID granularity.

Based on above observations, the following ENs can be resolved.

Editor’s note:	Whether and how the AIOTF chooses the RAN reader(s) is FFS.

Editor’s note:	AIoT RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Proposal 1: AIOTF performs AIoT RAN reader selection based on OAM configured A-IoT RAN information (supported Area, served reader ID list, location of reader) and target area information received in AF request.

Editor’s note:	It is FFS how to handle the case that the AF provides other information than target area information, for the purpose of reader selection. In such case how the NEF and/or AIOTF use it to select the RAN reader.

The AF can provide "the target area information" and/or "information about the target AIoT Device(s)". In addition, the last known reader might be known to the AIOTF handling the request. Therefore, we need to provide a precedence between information used to select readers.

If the AIOTF has been provided with “target area information” then this should always be used, as the AF has determined an area in which the operation should be performed. This may be used, when for example, at AF wants to check whether a Device (or devices) are within a certain area.

If the AIOTF has not been provided with “target area information”, then the AIOTF selects readers based on operation policy/AIOTF implementation. Part of this implementation may take into account any “last reader” information to optimise the procedure. Similar to paging strategies, we provide information which can be used, but do not specify how and when that information is used. The AIOTF should not limit selection to the last known reader, as if the AF wanted to target a single reader/area it could have provided that area in its request.

Proposal 2: Clarify the above AIoT RAN reader selection criteria.

Editor’s note:	How AIOTF selects the AMF and the AIoT RAN in the indirect path is FFS.

Proposal 3: The AMF capable of Ambient IoT can register its support of AIoT service as part of its NF profile in the NRF. The AIOTF can discover the AIoT capable AMF via NRF. The AIOTF selects the AMF to route the AIoT messages if the selected RAN/Reader is within the coverage of the AMF.

Editor’s note:	The AIOTF or AMF configuration of AIoT RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how AIoT RAN transfers and updates its information to the AIOTF in indirect path.

Proposal 4: FFS on RAN3 signalling for the A-IoT RAN information. Merge the two ENs.

Editor’s note:	How the NEF converts target area information to internal area information and the format of the internal area information is FFS.

The area where the service is performed is a key input to the request. However, the RAN reader information is not exposed to the AF. Then how to express the area where the service is performed is a question. Many other systems, for example WiFi management systems, allow grouping and those groups being the target of the operations (e.g. in the WiFi example APs located on the 3rd floor maybe a group and the same configuration applied to that group/APs, regardless of the exact model of AP).

Inspired by the WiFi management system,

-	1) the AIOTF can group readers and the AF makes a request targeting a group of readers. 5GC can determine from the targeted group to which readers are used for the AF request. The group of readers can represent the area where the service is performed or some other convenient grouping, based on AF and operator coordination to determine which readers are in which groups, e.g., based on the deployment location of readers to determine which group(s) they as part of. It is expected an identifier is assigned to the group for easy reference in the AIOTF and NEF service operations.

-	2) the AF can provide physical area and the 5GC can determine the target readers. In this case, the operator configures the 5GC with the mapping with the location of the reader, and the 5GC can use the physical area information to locate the reader that is going to handle the AF AIoT service request, e.g., using civic coordinates or shapes as described in TS29.572 or TS29.522.

An AF does not identify individual readers, or request individual readers to perform an operation. Instead, it requests either a group of readers or civic coordinates for an operation.

Proposal 5: AF can provide target area information in the format of a group identifier or Physical Area.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

5.3	AIoT Reader Selection

The AIOTF is responsible for AIoT Reader selection for the authorized inventory request or command requests it receives.

For AIoT RAN connectivity, the AIOTF selects NG-RAN node(s) and, optionally, a list of the RAN readers. The AIOTF selects the NG-RAN node(s) or RAN readers based on the AIoT RAN information held about for every NG-RAN node it can connect to and optionally its RAN readers. The held AIoT RAN information includes the NG-RAN's Supported Area, served RAN reader ID list, and optionally the location of each served RAN reader.

The AIOTF selects NG-RAN node(s) and, optionally, a list of the RAN readers based on the target area information included in the request it received. If the request did not include target area information, then the AIOTF determines target area information based on AIOTF implementation and operator policy, the AIOTF may further optimize the selection of NG-RAN nodes/RAN readers using available last known reader information in the local AIoT device context, if information about the target AIoT Device(s) is included in the request.

The target area information can be a group or geographical area. The AIOTF determines NG-RAN node(s) and/or RAN reader(s)s to use based on the target area information by:

-	If the target area information is a group, then the group identifies NG-RAN node(s), RAN reader(s), NG-RAN's Supported Area(s) or any combination of the aforementioned. If NG-RAN's Supported Area(s) are identified then the AIOTF further determines the NG-RAN node(s) and/or RAN readers from NG-RAN's Supported Area(s) using AIoT RAN information.

Which NG-RAN node(s), RAN readers and NG-RAN's supported Area(s) are a member of a group depends on the operator policy and implementation. NG-RAN node(s), RAN reader and NG-RAN's Supported Area(s) can be a member of zero or more groups.

NOTE X:	Group information is expected to be coordinated between the operator and the AF based on e.g., the deployment location of RAN readers, NG-RAN's Supported Areas and service level agreements, to allow the AF to determine which group to be used in a request.

-	If the target area information is geographical area, then the AIOTF:

1.	Converts the geographical area into NG-RAN's Supported Area based on local mapping configuration.

NOTE Y:	The mapping relationship between the geographical area and the NG-RAN's Supported Areas are determined e.g., based on the deployment location of AIoT RAN readers and NG-RAN's Supported Areas. And it is assumed that the AF provided geographical area can completely match to one or more of the NG-RAN’s Supported Areas.

2.	Determines the NG-RAN node(s) and, optionally, a list of the RAN readers from the NG-RAN's Supported Area(s) using AIoT RAN information.

Towards the selected NG-RAN node, the AIOTF sends the Inventory Request including requested Service Area Information directly or via the selected AMF.

-	If the RAN reader(s) are selected, the requested Service Area Information contains the target RAN reader ID list.

-	If the AIOTF receives geographical area as the target area information in the request, the requested Service Area Information also contains the converted NG-RAN’s Supported Area(s).

From the selected NG-RAN node, the AIOTF receives the Inventory Report including the RAN reader ID that represents the AIoT device’s location at reader ID granularity, the RAN reader ID is used by the AIOTF to update the last known RAN reader information in local AIoT device context.

The AF may provide the expected target area information in the AIoT service request. The NEF may convert the target area information received from the AF to internal area information.

Editor’s note:	It is FFS how AIoT RAN transfers and updates its information to the AIOTF over NGAP.

NOTE:	RAN reader information is not exposed to the AF.

# * * * * Second change * * * *

5.X	AIOTF Selection

The NEF is responsible for AIOTF selection for the authorized inventory request or command requests it receives.

The AIOTF registers itself in the NRF with its NF profile, this includes the

-	AIOTF ID/address,

-	NF type,

-	Service Area (Supported Area(s) of all connected NG-RAN(s) or Serving scope (e.g., geographical location)).

The NEF discovers and selects the AIOTF instances via NRF or local configuration based on the target area information included in the request it received. If the request did not include target area information, then the NEF may select the AIOTF based on available last known AIOTF information in target device profile data, NEF implementation and operator policy.

# * * * * Third change * * * *

5.Y	AMF Selection for indirect connectivity architecture

For indirect connectivity architecture (i.e., the AIOTF connects indirectly to AIoT RAN via an AMF), the AIOTF is responsible to select the AIoT capable AMF to relay the NGAP messages to/from the target NG-RAN nodes.

The AMF that is capable of Ambient IoT service can register its support of AIoT service as part of its NF profile in the NRF. If the AIOTF connects to AIoT RAN via an AMF, the AIOTF can discover the AIoT capable AMF via NRF, and then the AIOTF, based on local configuration, selects the AMF that connects the selected NG-RAN node(s)/RAN reader(s).

NOTE:	The local configuration contains the topologies between AMF and AIoT RAN, this is achieved via OAM configuration.

# * * * * Fourth change * * * *

5.Z	ADM Selection

The AIOTF is responsible for ADM selection. The following factors may be considered during the ADM selection:

-	AIoT Device Permanent ID,

-	local configuration,

-	3rd party related context.

# * * * * End of changes * * * *