3GPP TSG-SA2 Meeting #168	S2-2503863

# Stor-Göteborg, SE, 07th April 2025 – 11th April 2025								(was S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6269 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GS common time reference traceable to UTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN, IIoT |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is a mirror CR corresponding to a change in Rel-18. Current specification assumes the UPF/NW-TT and the gNB are time synchronized with the 5G internal system clock. The 5G internal system clock is however abstract as in real deployments the gNB deployments may be extremely distributed thus the 5G System entities (gNBs, UPF/NW-TTs) may not all have synchronized using the same epoch (origin of time). This causes an issue for residence time calculation in the UE/DS-TT or UPF/NW-TT. There is also no way for the UE to know the offset between the epoch used by the serving gNB and the serving UPF. In TS 38.401 clause 9.1 for gNB synchronization, there is a requirement that in case of TDD-unicast area not isolated, the common time reference shall be traceable to UTC. To addres the problem of the presence of multiple epochs in different user plane nores in the 5G system. This CR proposes to clarify in the specification that the 5G common time reference for all user plane nodes shall be traceable to UTC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that the user plane nodes are synchronized to the reference time shall have a common reference time that is traceable to UTC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uncertain support for residence time calculation where multiple epoch are used in gNB, NW-TT and DS-TT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.27.1.2	Distribution of timing information

##### 5.27.1.2.1	Distribution of 5G internal system clock

The 5G internal system clock shall be made available to all user plane nodes in the 5G system shall be traceable to UTC. The UPF and NW-TT may get the common time reference via the underlying PTP compatible transport network with mechanisms outside the scope of 3GPP. The common time reference shall be made available to UE with signalling of time information related to absolute timing of radio frames as described in TS 38.331 [28]. The common time reference shall be made available to DS-TT by the UE.

# * * * * End of changes * * * *