3GPP SA WG2 Meeting #168	S2-2503864

Gothenburg, Sweden, 07-11 April 2025

# Title:	<draft> LS Reply on support for multiple non-3GPP device identifiers

# Response to:	C1-250822

# Release:	Release 19

# Work Item:	UIA_ARC

Source:	SA2

To:	CT1

Cc:	CT3, CT4, SA3

Contact Person:

Name:	Abhijeet Kolekar

Tel. Number:

E-mail Address:	Abhijeet DOT kolekar AT intel DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 thanks CT1 for the liaison (C1-250822) seeking clarification on the Release-19 support of multiple non-3GPP device identifiers. SA2 appreciates CT1’s work on the NAS protocols for this feature and provides the following clarifications:

Q1: Whether the multiple non-3GPP device identifiers are allowed in a single PDU SESSION MODIFICATION REQUEST message?

Answer: Yes, the architecture allows multiple non-3GPP device identifiers to be included in a single PDU SESSION MODIFICATION REQUEST. SA2 confirms that a UE or 5G-RG may send more than one Non-3GPP Device Identifier in one request message when, for example, multiple tethered devices begin using the PDU session or need to be reported simultaneously. There is no restriction in TS 23.501 against carrying multiple identifiers at once – in fact, TS 23.501 envisages the SMF managing a set of device identifiers per PDU session. The NAS PDU Session Modification procedure should therefore support including an IE (or IEs) for each device identifier that is being added or removed. SA2 also notes that if the UE only has one device change at a time, it may, of course, send a single identifier, but combining them is allowed and expected for efficiency.

Q2: For the Non-3GPP device identifier not available scenario, whether the SMF rejects the Non-3GPP device identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message?

Answer: If the SMF (in coordination with PCF/UDR) finds that a provided Non-3GPP Device Identifier cannot be resolved or is unavailable (for instance, not recognized in subscription data or policy database), then the SMF should reject the entire PDU Session Modification request – rather than partially accept – in the current design. In other words, the modification procedure fails as a whole if any requested identifier cannot be handled. SA2 concurs with CT1’s approach to use a dedicated 5GSM cause value to indicate this scenario.

Q3: For the scenario of removal of more than one non-3GPP device identifier from the subscription in the UDR, whether a single or multiple non-3GPP device identifiers should be permitted in a single PDU SESSION MODIFICATION COMMAND message?

Answer: SA2 confirms that if the network needs to instruct the UE to remove multiple non-3GPP device identifiers, it may do so in a single PDU SESSION MODIFICATION COMMAND message. The downlink Command message can mirror the Request by including multiple identifier IEs (with an indication that they are to be removed).

2. Actions:

To SA and CT:

ACTION: 	Take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #169		19-23 May 2025	Fukuoka, Japan

TSG-SA2 Meeting #168		25-29 August 2025	Goteborg, Sweden