SA WG2 Meeting #168	S2-2503878

Goteborg, Sweden, 07 – 11 April 2025

Source:	Samsung

Title:	Architectural Assumptions and Principles

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The paper proposes Architectural Assumptions and Principles of the study.

# 1. Discussion

SA plenary have approved FS_5GSAT_Ph4_ARC SID. This paper proposes architecture assumptions and principles for the study.

# 2. Proposal

It is proposed to capture the following changes into TR 23.700-19.

* * * * Start of Changes * * * *

# 4	Architectural Assumptions

The following architecture assumptions are applied to the study:

-	The 5GC architecture for satellite access for NR as defined in TS 23.501 [a] is used as a baseline.

-	The EPC architecture for satellite access for IoT as defined in TS 23.401 [b] and the architecture enhancements to facilitate communications with packet data networks and applications as defined in TS 23.682 [c] are used as a baseline.

-	Impacts to UE, network functions and entities are minimised. To the extent possible, existing procedures and functionality is reused.

-	Inter-Satellite Links (ISL) and Feeder link are assumed to act only as transport layer links and are not specified in 3GPP, and the use of ISL is assumed to have no impact on the reference points between core network entities when these core network entities are onboard.

-	NR UE-Satellite-UE communication assumes a connection to the ground is always available.

-	NR UE-Satellite-UE communication assumes only non-IMS services (e.g., 5G-LAN) with IP and Ethernet PDU Session type are considered.

-	The enhancement for NR UE-Satellite-UE communication assumes only RAN and UPF, is present onboard of the satellites.

-	NR UE-Satellite-UE communication assumes that the serving satellite will be of MEO or of LEO type.

-	NR UE-Satellite-UE communication assumes communication between two UEs under the coverage of the same satellite or different satellites.

-	Solutions for NR UE-Satellite-UE communication are expected to have no UE impacts.

-	Solutions for IMS voice calls using GEO satellite access, only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 is not in the scope of the study.

-	The solutions for IMS voice calls using GEO satellite access, is assumed via NB-IoT access using EPC core only.

-	Solutions for IMS voice calls using GEO satellite access are not expected to have mobility enhancement.

-	Solutions for IMS voice calls using GEO satellite access, should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

-	Depending on operators policies, IMS enhancements defined for IMS voice calls using GEO satellite access may also be used in 5GS with NR NTN, but no additional study will be conducted in this study item.

* * * * End of Change* * * *