3GPP TSG-SA2 Meeting #83 	S2-110985

# 21 - 25 February 2011, Salt Lake City, Utah, USA

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.203 | CR | 0514 | rev | 1 | Current version: | 11.0.1 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifiction of ADC rules ACK in the PCRF-init IP-CAN session Modification Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SAPP |  |  |  |  |  |  | Date: |  | 02/02/2011 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | To align with IP-CAN session establishment/modification(PCEF), the TDF shall report the acknowledge of ADC rule to the PCRF and may indicate policy enforcement actions support in case some of the enforcement actions, required by PCRF, are not supported. However, this Ack message is missing in the PCRF-init IP-CAN session modification which still needs to state it. Moreover, same ack mechanism was also applied at PCEF for PCC provision in step 6/12. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the Ack message of ADC rule enforcement to the PCRF in the section 7.4.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It leads PCRF unaware of enforecment result of provisioned ADC rule and may lead faults in the further steps. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****** FIRST MODIFIED SECTION ********

7.4.2	IP-CAN Session Modification; PCRF initiated

This clause describes the signalling flow for the IP-CAN Session modification initiated by the PCRF. The AF may be involved. An example of the scenario is initiation and authorization of a session-based service for which an IP-CAN Session is modified. IP-CAN Session handling and handling of PCC rules for non-session based services, and also general handling of PCC rules that are not subject to AF-interaction is also applicable here.

# Figure 7.5: IP-CAN Session Modification; PCRF initiated

This procedure concerns both roaming and non-roaming scenarios. In the roaming case when home routed access applies (figure 5.1-3) or if case 2a applies (as defined in clause 7.1) for Local Breakout (figure 5.1-4), when a Gateway Control Session is used, the V-PCRF shall proxy Gateway Control and QoS Rules Request between the BBERF in the VPLMN and the H-PCRF over S9. For this case the H-PCRF may also initiate a Gateway Control and QoS Rules Provisioning procedure towards the BBERF in the VPLMN and proxy the information via the V-PCRF over S9.

For case 2b in the Local Breakout scenario (figure 5.1-4) and if the Gateway Control Session is terminated locally at the V-PCRF, the V-PCRF shall reply to/initiate Gateway Control Session and QoS Rules Request/Provisioning procedures locally without notifying the H-PCRF. For this case the V-PCRF shall proxy the Policy and Charging Rules Provisioning and Acknowledge over S9 between the PCEF in the VPLMN and the H-PCRF. If the AF is located in the VPLMN for this scenario, the V-PCRF shall proxy AF session signalling over S9 between the AF and the H-PCRF.

NOTE 1:	The case when the AF resides in the VPLMN is not showed in the figure.

In the non-roaming case (figure 5.1-1) the V-PCRF is not involved at all.

1a.	Optionally, the AF provides/revokes service information to the PCRF due to AF session signalling. The AF may subscribe at this point to notification of bearer level events related to the service information.

NOTE 2:	For the PCRF to generate the applicable events, the PCRF instructs the PCEF to report events related to the corresponding PCC rules. Such events are not shown in this sequence diagram.

1b.	Alternatively, optionally, in case of standalone TDF, the TDF detects the start/stop of a application traffic that matches with one of the activated ADC Rules. Then, in case of solicited application reporting, for the start of traffic detection, in case the enforcement actions were provided as a part of ADC rules, the TDF shall enforce those actions.

For the solicited application reporting, if the start/stop of application traffic detection event trigger was received from PCRF, the TDF shall provide application information to the PCRF, including the ADC Rule Identifier, start or stop of application traffic detection event trigger and the service flow descriptions, if available.

For the unsolicited application reporting, the TDF shall provide application information to the PCRF, including the ADC Rule Identifier and the service data flow descriptions, if available.

2.	The PCRF stores the service information if available and responds with the Acknowledgement to the AF. This is applicable to 1a case.

NOTE 3:	Without AF interaction, a trigger event in the PCRF may cause the PCRF to determine that the PCC rules require updating at the PCEF, e.g. change to configured policy.

NOTE 4:	This procedure could also be triggered by the Gateway Control and QoS Rules Request procedure as described in clause 7.7.3.

3.	The PCRF makes the authorization and policy decision.

4.	For the case 1b, the PCRF stores the service information if available and responds with the Acknowledgement to the TDF. If the TDF is standalone, for the solicited application reporting, the PCRF may provides a new ADC decisions to the TDF within this acknowledge. This may include ADC Rules activation, deactivation and modification. In case of local breakout, the V-PCRF shall provide functions to extract ADC rules from PCC rules provided by the H-PCRF over S9.

4a. If requested by PCRF, the TDF sends a Provision Ack (accept or reject of the ADC Rule operation(s)) to inform the PCRF about the outcome of the actions related to the decision(s) received in step 4.

5.	If there is no Gateway Control and QoS Rules Reply pending and there is a need to provision QoS rules, the PCRF initiates a Gateway Control and QoS Rules Provision Procedure as defined in 7.7.4 (applicable for cases 2a and 2b, as defined in clause 7.1).

If there are multiple BBERFs associated with the IP-CAN session, Step 4 is performed with the BBERFs that support UE/NW bearer establishment mode.

NOTE 5:	If there is a Gateway Control and QoS Rules Reply pending, e.g. this procedure was invoked from the Gateway Control and QoS Rules Request procedure as defined in clause 7.7.3, the PCRF shall use that opportunity for provisioning the applicable QoS rules. If there are multiple BBERFs associated with the IP-CAN session, and the procedure was invoked by a Gateway Control and QoS Rules Request procedure from the primary BBERF, the PCRF may receive a Gateway Control and QoS Rules Request from the non-primary BBERFs.

6.	The PCRF sends the Policy and Charging Rules Provision (PCC Rules, Event Trigger, Event Report) to the PCEF.

7.	The PCEF enforces the decision.

8.	If online charging is applicable, the PCEF may request credit for new charging keys from and/or shall return the remaining credit for charging keys no longer active to the OCS.

9.	If OCS was involved, the OCS provides the credit information to the PCEF, and/or acknowledges the credit report

10.	The GW (PCEF) may send an IP-CAN Bearer establishment, modification or termination request (applicable for case 1, as defined in clause 7.1).

An IP-CAN bearer modification is sent by the GW (PCEF) if the QoS of the IP-CAN bearer exceeds the authorized QoS provided by the PCRF in step 3.

An IP-CAN bearer termination request is sent by the GW (PCEF) if all PCC rules for an IP-CAN bearer have been removed.

11.	The GW (PCEF) receives the response for the IP-CAN Bearer modification or termination request (applicable for case 1).

12.	The PCEF sends Acknowledge Policy and Charging Rules Provisioning (accept or reject of the PCC rule operation(s)) to the PCRF.

13.	If the AF requested it, the PCRF notifies the AF related bearer level events (e.g. transmission resources are established/released/lost).

14.	The AF acknowledges the notification from the PCRF.