3GPP TSG SA WG2 Meeting #83	TD S2- 110949

21 - 25 February 2011, Salt Lake City, Utah, USA

Source:	AT&T

Title:	Clarification of High Level Requirements

Document for:	Approval

Agenda Item:	9.1

Work Item / Release:	NWK_PL2IMS / Rel. 11

Summary

This contribution proposes changes to draft TR 23.842 v030 to more precisely and fully state requirements. Without well-specified requirements, there is a risk of recommending a solution that does not meet the actual needs.

Proposals

Add material in sections 5.2 and 5.3 on requirements, as identified below.

Beginning of First Change

## 5.2	High Level Requirements

-	It shall be possible for the Cell-ID / PLMN ID to be provided to the IMS for the purpose of charging, lawful interception and retention of location information and potentially PSAP selection to be determined by the network. The information shall be provided for the originating UE in the originating record, and for the terminating UE in the terminating record.

-	For charging purpose –both online and offline, the CellID shall be provided corresponding to the serving cell at the beginning of chargeable communication session or event.

- 	For PSAP selection in the case of an IMS emergency call, the provision of the Cell-ID shall be based on functions specified in TS 23.167 [3].

-	The solution shall minimize the additional signalling.

-	The solution shall not require modifications of the UE.

-	The solution shall allow for the network to provide Cell-ID / PLMN ID if the UE has insufficient credentials (e.g. UICC-less case).

-	The solution for providing the Cell-ID / PLMN ID of the UE to the IMS shall not degrade the service level, e.g. session setup delay, compared to the degradation resulting from providing location in the corresponding CS service and shall not be perceptible to the end user.

-	The solution should allow for the provision of Cell ID/PLMN ID to IMS nodes for IMS subscribers using ICS in IMS Centralized Services scenarios as specified in TS 23.292 [4].

-	The number IMS entities required to do the retrieval of the Cell-ID /PLMN ID should be minimized.

End of First Change