3GPP TSG-SA3 Meeting #102Bis-e 	S3-210830

- e-meeting, 1 - 5 March 2021

- Source:	KPN

- Title:	Updated Solution #23: Initial key with validity time

- Document for:	Approval

Agenda Item:	2.9

# 1	Decision/action requested

It is requested to approve this update to Solution #23: Initial key with validity time

# 2	References

[1]	3GPP TR 33.847 Study on security aspects of enhancement for proximity based services in the 5G System (5GS), version 0.4.0

# 3	Rationale

This contribution updates Solution #23 (Initial key with validity time) to provide evaluation of the solution.

# 4	Detailed proposal

START OF CHANGE 1

## 6.23	Solution #23: Initial key with validity time

### 6.23.1	Introduction

This solution addresses Key Issue #12 Security of one-to-one communication over PC5. This solution assumes that the security context is established based on keys derived from a shared initial key provided by the 5GC. In order to prevent unbounded direct communication between ProSe UEs, the initial key and keys derived from it should have a validity time.

### 6.23.2	Solution details

The solution involves the following steps:

1.	A shared initial key is provisioned to  a selected set of authenticated and authorised ProSe UEs by the 5GC. This initial key has a validity time.

NOTE 1:	The provisioning of the shared initial key can occur when ProSe UEs are authenticated/authorized or when ProSe UEs request access to perform ProSe direct communication.

2.	As long as the initial key is valid, it can be used to derive keys needed for the security context. This security context enables establishment of a secure link between ProSe UEs with confidentiality and integrity protection.

NOTE 2:	Derivation of keys from initial key can take place even when the ProSe UEs are outside the coverage area.

NOTE 3:	Lifetime of the initial key and security context of a PC5 communication link are synchronised. Upon expiry of the initial key, the corresponding security context will also expire.

3.	Upon expiration of the intial key, all keys derived from it will become invalid. In order to continue secure communication ProSe UEs need to request for a new initial key from the 5GC. Until it gets the new initial key, ProSe UEs cannot perform direct communication with peer ProSe UEs.

### 6.23.3	Evaluation

This solution addresses the following security requirement of key issue #12: "The system shall support means for a secure refresh of the UE security context.". It provides means for a secure refresh of the UE security context.

The benefits of this solution are that it prevents unbounded direct communication between ProSe UEs and secure direct communication between ProSe UEs is possible even when the ProSe UEs are out of coverage.

The drawback of the solution is that once the initial key expires, ProSe UEs cannot have direct communication. They need to contact the 5GC to obtain a new initial key.

Editor’s Note: If and how this solution needs to be combined with other solutions is FFS.

END OF CHANGE 1