3GPP TSG-SA3 Meeting #102Bis-e 	S3-210836-r1

- e-meeting, 1st – 5th March 2021	Revision of xxxx

- Source:	Interdigital

- Title:	Conclusion for Key Issue #4

- Document for:	Approval

Agenda Item:	2.7

# 1	Decision/action requested

It is proposed to approve this pCR in TR 33.854.

# 2	References

# 3	Rationale

It is proposed to add a conclusion on Key Issue #4

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.854.

***	BEGIN OF CHANGES	***

# 7	Conclusions

Editor’s Note: This clause will contain the conclusion of the TR

## 7.4	Conclusion on KI #4

For key issue #4 on Location information veracity and location tracking authorization:

- Solution #6, solution #8 and solution#12 are chosen as basis for normative work, based on the following key common principles:

- The UAS NF (aka UFES) receives location request from USS/UTM. If authorized, UAS NF provides USS/UTM with UAV location information including the 3GPP UAV ID (GPSI) and optionally CAA-level UAV ID (in the case of "unknown" UAV tracking).

- To obtain UAV location information, the UAS NF uses current location services (LCS) as supported by AMF or GMLC. When requesting location, the UAS NF uses provided 3GPP UAV ID if provided by USS/UTM.

- The UAS NF ensures that the USS/UTM is authorized to track the location of a given UAV before sending the UAV location information to USS/UTM.  UAS NF checks that the USS/UTM is associated with the UAV from a prior UUAA procedure.

***	END OF CHANGES	***