3GPP TSG-SA3 Meeting #102Bis-e 	S3-210838

- e-meeting, 1st – 5th March 2021	Revision of xxxx

- Source:	Interdigital

- Title:	Conclusion for Key Issue #2

- Document for:	Approval

Agenda Item:	2.7

# 1	Decision/action requested

It is proposed to approve this pCR in TR 33.854.

# 2	References

# 3	Rationale

It is proposed to add a conclusion on Key Issue #2

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.854.

***	BEGIN OF CHANGES	***

# 7	Conclusions

Editor’s Note: This clause will contain the conclusion of the TR

## 7.2	Conclusion on KI #2

For key issue #2 on Pairing authorization for UAV and UAVC:

- For 5GS: solution #5, solution #11, solution#14, solution #15 are chosen as the basis for normative work, based on the following key common principles:

- Pairing authorization is performed after the UAV has been successfully authenticated/authorized by USS/UTM as per conclusion for key issue#1.

- If UAV authentication and authorization by USS/UTM is performed during a PDU Session establishment procedure and if a common PDU Session is used for both USS/UTM and C2 communication, pairing authorization may be performed as part of that common PDU Session establishment.

- The UAV provides in the PDU Session establishment/modification request an indication that the request is for C2 communication, and aviation pairing information which contains a CAA-level UAV ID, peer UAV-C information and security information identifier.

- The SMF triggers the pairing authorization procedure with USS/UTM via a UAS NF (aka UFES), providing transparently the aviation pairing information to USS/UTM.

- The SMF receives from the USS/UTM connectivity authorization information (e.g., UAV-C IP address) for pairing authorization. The SMF uses that information to set the proper traffic forwarding rules in the PDU Session to enable C2 communication with UAV-C. The SMF receives from the USS/UTM aviation pairing authorization information which it sends to the UAV in a NAS SM message. Authorization information contains: a CAA-level UAV ID, security information.

- USS/UTM may trigger new pairing authorization (e.g., for a change of UAV-C) via UAS-NF providing new aviation/connectivity authorization information. The SMF handles that information in a similar fashion as for initial pairing authorization and provides UAV with the received pairing authorization information along with pairing authorization indication and cause as UAV-Change respectively.

- USS/UTM triggers revocation of pairing authorization via UAS-NF which notifies the SMF. The SMF performs the applicable procedures in response to the revocation notification (e.g., PDU Session modification or release if a dedicated PDU Session for C2 communication is used) by providing pairing revocation indication and pairing revocation information (i.e., CAA level UAV ID and UAV-C ID) to the UAV accordingly.

- For EPS: solution #13 is chosen as the basis for normative work, with similar principles as for 5GS above applied to SMF+PWG-C and PDN connection.

***	END OF CHANGES	***