3GPP TSG-SA3 Meeting #102Bis-e 	draft_S3-210846-r2

- e-meeting, 1 – 5 March 2021												Revision of S3-210846

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Evaluation of solution #2.2 Encryption of authentication failure message types by UE with new keys derived from K_AUSF

- Document for:	Approval

Agenda Item:	2.5

# 1	Decision/action requested

Evaluation of solution #2.2 Encryption of authentication failure message types by UE with new keys derived from K_AUSF

# 2	References

[1]	3GPP TR 33.846

[2] 	3GPP TS 33.501

# 3	Rationale

This contribution proposes an additional evaluation for solution #2.2 of 3GPP TR 33.846 [1].

Solution #2.2 proposes a unique format of encryption of authentication response message with new keys derived from KAUSF.The following table discusses issues/limits seen with this solution.

| Scenario | Topic | Issue with solution |
| --- | --- | --- |
| Case 1 | KAUSF used in UE and AUSF | Issue 1: KAUSF in UE and AUSF is different and so decryption at AUSF will fail. Reason: In AUSF, during initial 5G AKA challenge, Key KAUSF is stored in AUSF (received as part of Home Environment Authentication vector). In UE, for authentication failure case, USIM will not generate CK, IK, so KAUSF at ME can’t be generated. As mentioned in the solution, KAUSF will be zero at UE side. AUSF uses Key KAUSF (valid value) received from UDM and UE uses KAUSF as zero. So, the generated encryption keys KE and integrity keys KM will be different at UE and AUSF. So decryption at AUSF will fail. |
| Case 2 | Success case of Authentication | Issue 2: In case of success, AMF doesn’t verify the RES*. Serving network level verification is removed with this proposal. Reason: Authentication failure message is used in case of failure and Authentication response message is used in case of success. For success case, Authentication response message proposed in this solution will share RES* in RES_DATA and SEAF / AMF can not verify HRES* with HXRES* because complete content of Authentication response is encrypted and only AUSF can decrypt it. No information provided between steps 4 to 7 about the RES* verification at AMF. |

# 4	Detailed proposal

It is proposed that SA3 review and approve the following text to 3GPP TR 33.846 [1] to update the evaluation of Solution #2.2: Encryption of authentication failure message types by UE with new keys derived from K_AUSF.

Whether the editor’s note can be removed with these additions can be decided during the meeting.

*** START of CHANGE ***

#### 6.2.2.3	Evaluation

Editor's Note: Further evaluation is needed.

The protection of SQN will only work over 5G network.

- The solution has impact on the serving network when 5G AKA is used.

The solution requires changes to the serving network for 5G AKA.

KAUSF used in UE and AUSF are not in sync during Authentication failure procedure: In AUSF, during initial 5G AKA challenge, Key KAUSF is stored in AUSF (received as part of Home Environment Authentication vector). But according to the solution, in UE, for authentication failure case, USIM will not generate CK, IK, so KAUSF at ME cannot be generated. As mentioned in the solution, KAUSF will be zero at UE side. AUSF uses KAUSF (valid value) received from UDM, but UE uses KAUSF as zero. So, the generated encryption keys KE and integrity keys KM will be different at UE and AUSF. So decryption at AUSF will fail.

Serving network verification of RES* is removed for 5G AKA, the AMF is not able to verify RES* against XRES*: Authentication failure message is used in case of failure; Authentication response message is used in case of success. For success case, Authentication response message proposed in this solution will share RES* in RES_DATA. SEAF / AMF cannot verify HRES* with HXRES* because the complete content of Authentication response is encrypted and only AUSF can decrypt it. No information between steps 4 to 7 about the RES* verification at AMF is provided.

*** END of CHANGE ***