3GPP TSG-SA3 Meeting #102bis-e 	draft_S3-210849-r1

- e-meeting, 1 – 5 March 2021													Revision of S3-210849

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Key issue risk assessment

- Document for:	Approval

Agenda Item:	2.5

# 1	Decision/action requested

Key issue risk assessment

# 2	References

[1]	TR 33.846

# 3	Rationale

In SA3#102-e, it was proposed to add a key issue risk assessment. The following is proposed to add to the table.

# 4	Detailed proposal

************ START OF CHANGES

### 7.0.1	Assessment of attack risk

Editor's note: This clause should provide for all KIs addressed a clear statement (in a condensed way) on the security threat, i.e. the likelihood of the attack to happen under which conditions, level of severity.

| Key Issues | Security threats | Comments (e.g likelihood, level of severity) |
| --- | --- | --- |
| Key Issue #2.1 – Linkability by distinguishing MAC failure and synchronization failure | Traceability of the user/victim | : Severity is high, and likelihood is high. |
| Key Issue #2.2 – SUCI linkablity by replay of SUCI | Traceability of the user/victim | Severity is low, and likelihood is low. |
| Key Issue #2.2 – SUCI linkablity by generating different SUCIs from same SUPI | Traceability of the user/victim | : Severity is low, and likelihood is low. |
| Key Issue #2.2 – SUCI based DoS attack | Dos attack on UDM. | Severity is low, and likelihood is low. DoS attack claimed by the key issue is not so serious. Replaying of SUCI will not drain the processing power of UDM. No normative work needed. |
| Key Issue #3.1: Attack due to expired authentication result in the UDM | n/a | This KI was already concluded to be not addressed. |
| Key Issue #3.2: SUPI guessing attacks | Privacy threat | Severity is low, and likelihood is low. |
| Key Issue #4.1: Protection of SQN during AKA re-synchronisations | Privacy leakage of subscribers | Severity is high, and likelihood is high. |

************ END OF CHANGES