3GPP TSG-SA3 Meeting #102Bis-e 	draft_S3-210853-r1

- e-meeting, 1 - 5 March 2021																Revision of S3-200853

- Source:	Thales

- Title:	pCR to TR 33.846: Impacts of solutions

- Document for:	Approval

Agenda Item:	2.5

# 1	Decision/action requested

Impacts of solutions: update of the comparison table

# 2	References

[1]	3GPP TR 33.846 v0.10.0

# 3	Rationale

Addition of solutions in the comparison table of clause 7.0.2 on “Impacts of solutions”.

# 4	Detailed proposal

It is proposed that SA3 review and approve the following pseudo-CR to 3GPP TR 33.846 [1].

*** START of CHANGE ***

### 7.0.2	Impact of solutions

Table: Comparison table of UE and network impacts per solution

|  | UE impacts | Network impacts |  | comments |
| --- | --- | --- | --- | --- |
| Solutions |  | AMF/ SEAF | UDM/ ARPF |  |
| Solutions for resilience against identifier linkability |  |  |  |  |
| #2.1: Handling of Sync failure by AUTS encryption | ME | X | X | Risk of linkability attack. |
| #2.2: Encryption of authentication failure message types by UE with new keys derived from K_AUSF | Probably ME | X | X | Authentication failure message for the first UE registration is left unencrypted |
| #2.3: Unified authentication response message by UE | USIM | X | X |  |
| #2.4: MAC-S based solution | USIM | X | X |  |
| #2.5: Encryption of authentication failure message with SUCI method | USIM or ME | X | X | This solution relies on the availability of SUCI mechanism. |
| #2.6: Certificate based encryption of unicast NAS message | USIM and ME | X | X | This solution relies on the availability of PKI architecture, and there is companion list of open questions. |
| #2.7: Mitigation against the SUCI replay attack | USIM and ME | X | X |  |
| Solutions for availability aspects of SUCI usage |  |  |  | Usage of the long term key K for other uses. |
| #3.1: Mitigation of SUPI guessing and SUCI replay attack using long term key | USIM and ME | X | X | Security issue due to the use of the long term key K for other purpose than AKA. |
| Solutions on re-synchronisation in AKA |  |  |  |  |
| #4.1: Using MACS as freshness in the calculation of AK | USIM |  | X |  |
| #4.2: Using symmetric encryption function to protect SQN during a re-synchronisation procedure in AKA | USIM |  | X | This solution requires the definition of a new function f6* for AKA procedure. |
| #4.3: SQN protection by concealment with SUPI in USIM | USIM | X | X | Solution works if SUCI computation is performed in the USIM. |
| #4.4: SQN protection during re-synchronisation procedure in AKA | USIM and ME |  | X | Limitation of SQNMS leakage depends on RAND storage in the USIM. |
| #4.5: AUTS SQNMS solution for 5GS | USIM and ME | X | X | TBD |
| #4.6: Using time-based or partly time-based SQN generation |  |  |  | Solution does not work for non-time based SQN generation. Solution requires time based SQN generation |
| #4.7: SQN protection by concealment with SUPI with f5* | USIM and ME | X | X | TBD |

NOTE:	A mapping table on which solution is addressing which key issue is provided in clause 6.0.

*** END of CHANGE ***