- 3GPP TSG SA WG3 Meeting #102bis-e                            	S3-210899

- e-meeting, 1 – 5 March 2021	                         revision S3-21abcd

- Source:	Huawei, HiSilicon

- Title:	New Solution for Interworking from 5GS to EPS

- Document for:	Approval

Agenda Item:	2.3

# Decision/action requested

This contribution proposes to add a new solution in TR 33.853

# References

# Rational

This contribution adds a new solution for interworking from 5GS to EPS in TR 33.853.

# Detailed proposal

*************** Start of 1st Change ****************

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.801:"Study on new radio access technology: Radio access architecture and interfaces".

[3] 	3GPP TS 33.401: "5G System Architecture Evolution (SAE); Security architecture".

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[5]	S3-181429, LTE and the upcoming 5G standard (GSMA).

[6]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[7]	3GPP TS 38.413: " NG-RAN; NG Application Protocol (NGAP)".

[8]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[9]	3GPP TS 36.331: "Radio Resource Control (RRC); Protocol specification".

[10]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[11]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[12]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[yy]	3GPP TS 23.502: “Procedures for the 5G System (5GS); Stage 2”.

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

## 6.X	Solution #X: Interworking from 5GS to EPS

### 6.X.1	Introduction

The solution addresses key issue #7. The solution provides a whole picture for UP security policy handling of interworking from 5GS to EPS.

### 6.X.2	Network options affected

This solution is applicable to the following network options:

- Option 1 – eUTRA with EPC

- Option 2 – NR standalone with 5G Core

- Option 3 – EPC based Dual Connectivity of eUTRA and NR RAT

- Option 4 – 5G core based Dual Connectivity (NR master – eUTRA secondary)

- Option 5 – 5G core with eUTRA

- Option 7 – 5G core based Dual Connectivity (eUTRA master – NR secondary)

### 6.X.3	Solution description

Currently, there is restriction for interworking as depicted in clause 5.10.3 in TS 23.501 [12],

“For PDU Sessions with UP integrity protection of UP Security Enforcement Information set to Required, the SMF shall not trigger the EPS bearer ID allocation procedure in TS 23.502 [3] clause 4.11.1.4.”,

It means that when one of the PDU session’s UP integrity protection policy is set to “Required”, this PDU session cannot be interworked from 5GS to EPS because no EPS bearer ID is allocated.

As depicted in clause 4.11.5.3 in TS 23.502 [yy],

“Step 4: If the EPS Interworking indication received from AMF indicates that the UE supports EPS interworking and the SMF determines, based on the EPS interworking support indication from the AMF and additional UE subscription data (e.g. whether UP integrity protection of UP Security Enforcement Information is not set to required, EPS interworking is allowed for this DNN and S-NSSAI), that the PDU Session supports EPS interworking, the PGW-C+SMF FQDN for S5/S8 interface is included in the Nudm_UECM_Registration Request.”,

it means that when one of the PDU session’s UP integrity protection policy is set to “Required”, this PDU session cannot be interworked from 5GS to EPS because SGW cannot find PGW-C+SMF with its FQDN for S5/S8 interface.

However, if UP integrity protection is supported in EPC, such restriction should not be valid. Thus, this solution proposes as below:

-	When the UE is in 5G, if the network knows that the UE supports user plane integrity protection with an eNB, the restriction for interworking shall be cancelled. And the PGW-C+SMF also prepares mapped UP security policy for EPS, and stores it in EPS bearer context.

-	During or after 5GS to EPS interworking procedure, e.g. 5GS to EPS idle mode mobility, 5GS to EPS handover, since PGW-C+SMF has already stored UP security policy in EPS bearer context, the new MME can retrieve EPS bearer context with UP security policy from PGW-C+SMF via AMF transparently.

-	The eNB can retrieve UP security policy from the MME, and use AS SMC and RRC Reconfiguration procedure to configure the usage of UP Integrity Protection with the UE.

#### 6.X.3.1	Impacts to 5GC procedures

During Registration procedure, the AMF receives indication that the UE supports user plane integrity protection with an eNB from the UE, and stores it in the UE context.

During the PDU session establishment procedure enhanced for interworking, the procedure is the same as the procedure depicted in clause 4.11.5.3 in TS 23.502 [yy] with the following modifications:

-	In step 3, if the AMF knows UE support UP IP with eNB according to the UE context, the AMF sends the indication that the UE supports user plane integrity protection to the SMF+PGW-C.

-	In step 4, the SMF+PGW-C determines whether restriction of EPS interworking is still available based on the received indication, i.e. if the indication is received, the SMF+PGW-C can provide PGW-C+SMF FQDN for S5/S8 interface to the UDM without considering the UP integrity protection.

-	After step 4, if the indication is received, the PGW-C+SMF can trigger EPS Bearer ID allocation procedure as depicted in clause 4.11.1.4.1 in TS 23.502 [yy] without considering the UP integrity protection.

-	After step 4, if the indication is received, the PGW-C+SMF also maps UP security policy from PDU session to EPS bearer, and stores it in EPS bearer context.

#### 6.X.3.2	5GS to EPS Idle Mode Mobility

The procedure is the same as the procedure depicted in clause 4.11.1.3.2 in TS 23.502 [yy] with the following modifications:

-	In step 5c and 6, since the SMF+PGW-C already stored UP security policy in EPS bearer context, the SMF+PGW-C sends UP security policy in the EPS bearer context to the AMF, and the AMF sends the EPS bearer context to the MME transparently.

After receiving the UP security policy, the MME maps the UE security policy from EPS bearer to E-RAB, and   sends it to the eNB during any existing procedures, e.g. attach, service request, bearer management procedure, etc.

#### 6.X.3.3	5GS to EPS Handover

The procedure is the same as the procedure depicted in clause 4.11.1.2.1 in TS 23.502 [yy] with the following modifications:

-	In step 2c and 3, since the SMF+PGW-C has already stored UP security policy in EPS bearer context, the SMF+PGW-C sends UP security policy in the EPS bearer context to the AMF, and the AMF sends the EPS bearer context to the MME transparently.

-	In step 6, the MME maps the UE security policy from EPS bearer to E-RAB, and sends it to the eNB in the Handover Request message.

Editor Note: It is FFS to decide whether this solution can be adopted based on the decision which entity provides UP IP policy, MME or PGW-C+SMF.

### 6.X.4	Solution evaluation

The solution impacts UE, eNB, MME, AMF and SMF+PGW-C.

*************** End of 2nd Change ****************