- 3GPP TSG SA WG3 Meeting #102bis-e                            	S3-210907

- e-meeting, 1 – 5 March 2021	                         revision S3-21abcd

- Source:	Huawei, Hisilicon

- Title:	New Solution for Security for Provisioning of SNPN Credentials with initial credential

- Document for:	Approval

Agenda Item:	2.12

# Decision/action requested

This contribution proposes to add a new solution in TR 33.857

# References

# Rational

This contribution adds a new solution for key issue #2 in TR 33.857.

# Detailed proposal

*************** Start of 1st Change ****************

## 6.X	Solution #X: Security for Provisioning of SNPN Credentials with initial credential

### 6.X.1	Introduction

The solution addresses key issue #2, especially for provisioning SNPN credentials in case that the UE and the PS can be pre-configured with initial credentials.

### 6.X.2	Solution details

The assumptions include:

- UE and PS have been pre-configured with initial credentials used for provisioning authorization and protection.

- O-SNPN and SO-SNPN have been involved in primary authentication based on default credential, e.g. as depicted in solution 12.

The detailed procedure is:

After successful primary authentication based on default credential between UE, O-SNPN and SO-SNPN, UE and O-SNPN have maintained NAS key.

The UE requests to establish a restricted PDU session, the SMF in SO-SNPN gets preconfigured PS identity and triggers secondary authentication between the UE and the PS. After successful secondary authentication, the PS identity is included in PDU Session Establishment Response message, and is sent to the UE. Otherwise, the PDU session establishment procedure is cancelled. Besides, the SMF in SO-SNPN notifies authentication result to UDM in SO-SNPN, and the UDM updates UE’s subscription. The UE can only access the PS in the restricted PDU session. This procedure is protected based on NAS key.

The UE and the PS use initial credential to establish a secure tunnel to protect the following provisioning. Then, subscription on both UE and UDM side has been synchronized.

Editor’s Note: Details for initial credential is ffs.

Editor’s Note: Details and figures clarifying the steps are needed.

Editor’s note: It is FFS to clarify what are the credentials used to perform the secondary authentication.

### 6.X.3	System Impact

UE, SMF, and UDM are impacted.

PS may be impacted, if the PS does not support EAP authentication.

### 6.X.4	Solution evaluation

TBA

*************** End of 1st Change ****************