- 3GPP TSG-SA3 Meeting #102bis-e 	draft_S3-210913-r1

- e-meeting, 1 – 5 March 2021

- Source:	Huawei, Hisilicon

- Title:	F1 interface security for IAB in NR-DC mode

- Document for:	Approval

Agenda Item:	2.18

# 1	Decision/action requested

This contribution proposes to update KI for IAB TR 33.824.

# 2	References

[1]	RAN3_110-e_agenda_with_Tdocs20201112_EOM.zip https://www.3gpp.org/ftp/tsg_ran/WG3_Iu/TSGR3_110-e/Inbox/Chairmans_Notes/RAN3_110-e_agenda_with_Tdocs20201112_EOM.zip

# 3	Rationale

In RAN WG, there is an ongoing WID on IAB in R17. In RAN3, it’s agreed to define the CP-UP separation via separate NR access link. This enhancement defines the analogue of F1 routing for standalone, i.e., for IAB-nodes that use NR-DC instead of EN-DC. The IAB-donor could be Master Node or Secondary node. The following context in the chairman note are agreed:

In Rel-17 eIAB, the following two scenarios are supported for CP-UP separation:

- Scenario 1: F1-C uses NR access link via M-NG-RAN node (non-donor node) + F1-U uses backhaul link via S-NG-RAN node (donor node)

- Scenario 2: F1-U uses backhaul link via M-NG-RAN node (donor node) + F1-C uses NR access link via S-NG-RAN node (non-donor node)

In SA3, security mechanisms for F1 interface between the IAB-node (gNB-DU) and the IAB-donor-CU is already defined for SA mode and EN-DC mode. It’s needed to study the security features for NR-DC mode.

# 4	Detailed proposal

It is proposed to approve the following changes in IAB TR.

***	BEGIN OF 1st CHANGE	***

- 5.3.X	Key issue #3.X: F1 interface security for IAB in NR-DC mode

#### 5.3.X.1	Key Issue details

The IAB realized in SA mode or EN-DC is already supported as descripted in clause 5.3.1.1. An IAB donor node has gNB-CU functionality and another IAB node has gNB-DU functionality. Each IAB node is connected to the IAB-donor node either directly (i.e., a single hop) or via another IAB node (i.e., multiples hops).

In addition, NR-DC is also specified to support the CP-UP separation via separate NR access link. The following two scenarios as shown in Fig. 5.3.X.1 (a) and (b) are supported for CP-UP separation:

- Scenario 1: F1-C uses NR access link via M-NG-RAN node (non-donor node) + F1-U uses backhaul link via S-NG-RAN node (donor node)

- Scenario 2: F1-U uses backhaul link via M-NG-RAN node (donor node) + F1-C uses NR access link via S-NG-RAN node (non-donor node)

(a) Scenario 1                                                            (b) Scenario 2

Figure 5.3.X.1: Scenarios for CP-UP separation

F1 interface security for the wireline link between CU and DU is specified in TS 33.501[5]. Dynamic PSK computation for IKEv2 PSK authentication may also be supported. When dynamic PSK is used, the IAB-node and the IAB-donor need to calculate the PSK (KIAB). However, the KIAB generation function is only defined for SA mode and EN-DC mode. The KIAB generation function for NR-DC mode is not clear. IAB-donor could be Master Node or Secondary Node. The input key to generate KIAB needs to be defined to keep consistency between IAB node and IAB-donor.

This key issue is to study potential solutions for F1 interface security for IAB in NR-DC mode.

#### 5.3.X.2	Security threats

If the input key to generate KIAB is not well-defined, the Kiab may mismatch between IAB node and IAB donor, which leads to the failure of F1 interface protection when KIAB is used in IKEv2 PSK authentication.

#### 5.3.X.3	Potential security requirements

The KIAB in IAB-node and IAB-donor shall be consistent if KIAB is used in IKEv2 PSK authentication for IAB in NR-DC mode.

***	END OF 1st CHANGE	***