- 3GPP TSG-SA3 Meeting #102bis-e	draft_S3-210995-r4

- e-meeting, 1 – 5 March 2021	revision of S3-210995

- Source:	CATT

- Title:	pCR to TR33.847- Update Solution#30

- Document for:	Approval

Agenda Item:	2.9

# 1	Decision/action requested

Approve the pCR below

# 2		Rationale

This contribution tries to remove the following Editor’s notes:

Editor’s note: The difference from LTE procedures needs to be highlighted.

See the introduction part.

Editor’s note: It is FFS whether this is a feasible solution under unprotected one-to-one PC5 communication when reuses unicast mechanism in TS 33.536 [8].

See step 2

Editor’s note: Whether and how the Remote UE may use its 5G native security context to connect via the Relay UE is FFS.

See step 1, 5G-GUTI can be a parameter in the request and 5G security context can be used in the procedure.

Editor’s note: The necessity of AV in step 5 & 6 is FFS.

Revision has be done, see step 5-7

Editor’s note: Whether AUSF can check authorization with UDM that Remote UE is authorized as Remote UE and based on Relay Service code is FFS.

Revision has be done, see step 8.

# 3	Detailed proposal

#### *** BEGIN CHANGES ***

## 6.30	Solution #30: UE-to-Network Relay security based on primary authentication

### 6.30.1	Introduction

This solution addresses the Key Issue #3, Key Issue #4 and Key Issue #9. It is applicable to both L2 and L3 UE-to-Network Relay architectures.

The solution reuses the PC5 unicast communication security procedure defined in TS 33.536 [8] for the PC5 communication security.

The main differences between this solution and the LTE procedure defined in TS 33.303 [6] are as follows:

-	In this solution, the PKMF in LTE is replaced by the AUSF of the Relay UE.

-	In this solution, 5GPRUK is derived from Kausf, but in LTE, PRUK is generated by the GBA system.

-	In LTE, the UE-to-network relay is only used for public safety services, but in this solution, it is mainly used for commercial services.

-	In LTE, the Remote UE provides PRUK ID or IMSI in a Direct Communication Request, but in this solution, the Remote UE provides 5G PRUK ID and/or 5G-GUTI/SUCI in a Direct Communication Request.

Editor’s note: It is FFS whether this is a feasible solution under unprotected one-to-one PC5 communication when reuses unicast mechanism in TS 33.536 [8].

### 6.30.2	Solution details

The UE-to-network relay security flow based on primary authentication is described as follows:

1.	The Remote UE generates a freshness parameter Nonce_1 for the one-to-one communication and sends a Direct Communication Request to the Relay UE. In addition to the one-to-one communication parameters, the message may contain the following parameters:

-	a Relay Service Code that the Remote UE would like to access.

-	a 5GPRUK ID (if the Remote UE has a 5GPRUK for this relay connection)

-	a 5G-GUTI (if the Remote UE has a 5G security context) or a SUCI (if the Remote UE does not have a 5G security context). In the case of 5G-GUTI, 5GPRUK ID, GUTI, Relay Service Code and Nonce_1 should be integrity protected with KNASint.

NOTE 1:	 5GPRUK and 5GPRUK ID are equivalent to PRUK and PRUK ID in TS 33.303 [6], respectively.

2.	The Relay UE sends a NAS Relay Key Request to its serving AMF. The message includes the parameters received from the Remote UE, i.e. 5GPRUK ID and/or SUCI/GUTI, Relay Service Code and Nonce_1. In case a 5G-GUTI is included, these parameters are integrity protected with KNASint by the Remote UE.

Editor’s note: It is FFS if the lack of integrity protection in case 5G-GUTI is not present has any security risks.

3.	The AMF of the Relay UE checks whether the Relay UE is authorized to be a Relay UE according to the Relay Service Code. The relay service authorization information is stored in the UDM of the Relay UE. In the case that the Direct Communication Request message of the Remote UE contains a 5G-GUTI, the AMF should retrieve the SUPI and current 5G security context of the Remote UE, and then check the integrity protection of the parameters of Direct Communication Request message as received in the NAS Relay Key Request message.

Editor’s note: How to handle the cases that 5G-GUTI is established with another AMF or anther PLMN is FFS.

Editor’s note: The UE context transfer procedure to retrieve the 5G security context from another AMF is FFS.

4.	The AMF of the Relay UE sends a Relay Key Request to the AUSF of the Remote UE. The message includes Remote UE's SUCI or SUPI, Relay Service Code, Nonce_1 and the 5GPRUK ID (if it exists).

5-7.	In the case of a SUCI in the Relay Key Request, the AUSF performs a primary authentication procedure defined in TS 33.501 [14] to the remote UE through the Relay UE.

8.	The AUSF of the Remote UE checks whether the Remote UE is authorized to be a Remote UE according to Remote UE’s SUPI and Relay Service Code. The relay service authorization information is stored in the UDM of the Remote UE.

9-10.	In the case of a 5GPRUK ID in the Relay Key Request, the AUSF of the Remote UE retrieves the 5GPRUK from the UDM using the Remote UE's SUPI and the 5GPRUK ID.

11.	If there is no 5GPRUK ID in the Relay Key Request or the 5GPRUK ID needs refreshing, a new 5GPRUK will be generated. The AUSF of the Remote UE derives 5GPRUK using Kausf and some other parameters, generates a corresponding 5GPRUK ID. The AUSF of the Remote UE should generate a 5GPRUK_Info from which the Remote UE can derive 5GPRUK and obtain 5GPRUK ID.

NOTE 2:	 The detailed structure of 5GPRUK_Info will be defined in normative phase.

12.	In the case that a new 5GPRUK is generated, the AUSF of the Remote UE stores the newly generated 5GPRUK and 5GPRUK ID in the UDM.

13.	The AUSF of the Remote UE generates a new random number as the 5GKD Freshness Parameter, and then generates a new 5GKD using 5GPRUK, 5GKD Freshness Parameter, Nonce_1, Relay Service Code etc.

NOTE 3:	 5GKD is equivalent to KD in TS 33.303 [6].

14-15.	The AUSF of the Remote UE sends 5GKD, 5GKD Freshness and 5GPRUK_Info to the AMF of the Relay UE, and then further passes them to the Relay UE.

16.	The Relay UE sends a Direct Security Mode Command to the Remote UE. In addition to one-to-one communication parameters, the message includes 5GKD Freshness Parameter and 5GPRUK_Info (if it exists).

NOTE 4:	 In Remote UE and Relay UE, 5GKD is equivalent to KNRP.

17.	In the case that there is a 5GPRUK_Info in the message, the Remote UE derives the 5GPRUK and obtains the 5GPRUK ID using the information in 5GPRUK_Info. The Remote UE stores the 5GPRUK and 5GPRUK ID. The Remote UE further derives the 5GKD and performs other procedure.

18.	The Remote UE sends Direct Security Mode Complete message to Relay UE.

Figure 6.30.2-1: UE-to-Network Relay security flow based on primary authentication

### 6.30.3	Evaluation

TBD

#### *** END OF CHANGES ***