3GPP TSG-SA3 Meeting #102bis-e	draft_S3-211002-r1

- e-meeting, 1 – 5 March 2021													Revision of S3-21xxxx

- Source:	Nokia, Nokia Shanghai Bell, Deutsche Telekom

- Title:	Annex on asymmetric delay attacks

- Document for:	Approval

Agenda Item:	2.10

# 1	Decision/action requested

Annex on attacks based on asymmetric delays

# 2	References

[1]	3GPP TR 33.851

# 3	Rationale

Two types of delay attacks are described in research papers and are proposed to be documented in this study.

# 4	Detailed proposal

******* START OF CHANGES

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501 "Security architecture and procedures for 5G System".

[3]	3GPP TS 23.501 "System architecture for the 5G System (5GS)".

[4]	3GPP TR 23.700-20 "Study on enhanced support of Industrial Internet of Things (IIoT) in the 5G System (5GS)".

[5]	IEEE 802.1Qcc "IEEE Standard for Local and Metropolitan Area Networks--Bridges and Bridged Networks -- Amendment 31: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[6]	IEEE 1588-2008 "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems".

[7]	RFC 7384 "Security Requirements of Time Protocols in Packet Switched Networks".

[8] 	Robert Annessi, Joachim Fabini, Felix Iglesias, and Tanja Zseby: "Encryption is Futile: Delay Attacks on High-Precision Clock Synchronization"; https://arxiv.org/pdf/1811.08569.pdf.

[9] 	Sergio Barreto; Aswin Suresh; Jean-Yves Le Boudec: "Cyber-attack on Packet-Based Time Synchronization Protocols: the Undetectable Delay Box"; published in: 2016 IEEE International Instrumentation and Measurement Technology Conference Proceedings; https://ieeexplore.ieee.org/document/7520408.

[10] 	Markus Ullmann; Matthias Vögeler: "Delay Attacks - Implication on NTP and PTP Time Synchronization"; published in: 2009 International Symposium on Precision Clock Synchronization for Measurement, Control and Communication; https://ieeexplore.ieee.org/abstract/document/5340224/.

******* NEXT CHANGE

Annex X (informative):
Asymmetric delay attacks

## X.1	Introduction

The possibility of an attacker delaying packets in one direction and by this introducing an asymmetric delay has been described in several research papers [8, 9, 10] and is also mentioned in [7]. This annex documents the issues with such attacks.

## X.2	Calculation of offset between clocks

Usually symmetric channel delays apply when PTP uses different event messages to synchronize the time between two ports, whereas each port is attached to a clock. This clock can be a boundary clock (which is usually a consuming TSN end station with 1 port) or an ordinary clock (which have at least 2 ports and forward the time) (5GS works as a transparent clock). Also, one of the clocks may be attached to a Grand master clock (which can be in 5GS or outside).

The calculation of the offset between clocks is based on the assumption that the channel delay in both directions (i.e. master to slave and slave to master) is symmetric (i.e. the time a message needs to traverse from one port to another is the equal). If this assumption holds, PTP time synchronization is highly accurate.

## X.3	Delay attacks on time synchronisation messages

An attacker having the possibility to delay packets, e.g. via ARP spoofing (Ethernet), BGP hijacking (IP) or a compromised in-path device or clock, may introduce an asymmetric delay (i.e. different delay for master to slave and slave to master). This delay may be deterministic or random.

There are two types of such asymmetric delay attacks: asymmetric selective message delay and asymmetric channel delay attacks:

-	In asymmetric selective message delay attacks, the attacker performs traffic analysis to identify specific synchronization messages to delay. Typically, the "Sync" message for master to slave or the "delay request" messages for slave to master are delayed. For this attack, an attacker must perform traffic analysis to identify the synchronization messages for delay.

- 	In asymmetric channel delay attacks, messages transmitted over the full channel are delayed in one direction. Due to this, the PTP offset calculation becomes wrong and the clocks could start to be not synchronized properly anymore.

The impact of these attacks may be DoS, accuracy degradation and false times being synchronized.

In contrast, if synchronisation messages are symmetrically delayed as described in clause X.2, an attacker delaying by purpose cannot degrade accuracy, because the same delay happens in both directions.

## X.4	Considerations

As reported in [8, 9, 10], encryption hardens the synchronization against selective message delay attacks, as it is more complex for an attacker to perform traffic analysis on the channel communication.

This annex documents the issue of asymmetric delay attacks. As 5GS works as a transparent bridge, the countermeasures are out of 3GPP scope.

******* END OF CHANGES