3GPP TSG-SA3 Meeting #102bis-e 	S3-211053r1

- e-meeting, 1 - 5 March 2021																Revision of S3-211053

- Source:	Qualcomm Incorporated

- Title:	Adding the revocation functionality to solution #14

- Document for:	Approval

Agenda Item:	2.7

# 1	Decision/action requested

The contribution proposes to add the revocation functionality to solution #14.

# 2	References

[1]	3GPP TR 33.854 v0.4.0

# 3	Rationale

Solution #14 has the revocation part missing and this contribution proposes to add that part of the solution.

# 4	Detailed proposal

It is proposed that SA3 approved the below pCR for inclusion in the TR [1].

**** START OF CHANGES ****

- 6.14.2.3	Revocation

Figure 6.14.2.3-1 shows how the authorisation for some connectivity can be revoked.

Figure 6.14.2.3-1: UAV connectivity revocation

1. The USS/UTM decides to revoke the UAV’s authorisation for some connectivity.

2. The USS/UTM sends an Authorisation Revoke request to the UFES including the 3GPP UAV ID and details of the connectivity (e.g. UAV-C’s IP address) to be revoked (e.g. a pairing with a UAV-C is no longer needed).

3. The UFES passes the Authorisation Revoke request to the relevant SMF(s) which are selected based on the details of the connectivity to be revoked.

Editor’s Note: SMF selection details are FFS.

4. The SMF removes the connectivity of the UAV based on the received details (e.g. prevents the UAV from communicating with the UAV-C). This is performed using PDU session release (e.g. when removing one of multiple PDU sessions) or PDU session modification (e.g. when restricting connectivity in the single PDU case).

Editor’s Note: It is FFS what information if any is provided to UAV.

5. The SMF confirms to the UFES that the revocation of connectivity has happened.

6. The UFES confirms to the USS/UTM that the revocation of connectivity has happened.

**** END OF CHANGES ****