3GPP TSG-SA3 Meeting #102bis-e 	S3-211060r1

- e-meeting, 1 - 5 March 2021																Revision of S3-2111060

- Source:	Qualcomm Incorporated

- Title:	Using TLS with AKMA to protect EDGE-1 and EDGE-4

- Document for:	Approval

Agenda Item:	2.8

# 1	Decision/action requested

The contribution propose to use TLS with AKMA to protect some Edge interfaces.

# 2	References

[1]	3GPP TR 33.839 v0.4.0

# 3	Rationale

This contribution proposes a solution that uses TLS with AKMA to protect EDGE-1 and EDGE-4.

# 4	Detailed proposal

It is proposed that SA3 approved the below pCR for inclusion in the TR [1].

**** START OF CHANGES ****

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.558: "Architecture for enabling Edge Applications."

[3]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in the 5G Core network (5GC)".

[4]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications".

[5]	3GPP TS 23.502: "Procedure for the 5G System; Stage 2".

[6]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[7]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[8]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[9]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[10]	Void.

[11]	3GPP TS 33.187: "Security aspects of Machine-Type Communications (MTC) and other mobile data applications communications enhancements".

[12]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[13]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[14] 	3GPP TS 23.501: "System Architecture for the 5G System".

[15] 	3GPP TS 23.003: "Numbering, addressing and identification".

[16]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[17]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[18]	IETF RFC 4279 "Pre-Shared Key Ciphersuites for Transport Layer Security (TLS)".

[19]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[20]	3GPP TR33.867: "Study on user consent for 3GPP services".

[21]	RFC 7858: "Specification for DNS over Transport Layer Security (TLS)".

[22]	RFC 8310: "Usage Profiles for DNS over TLS and DNS over DTLS".

[23]	3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[24]	IETF RFC 7616: "HTTP Digest Access Authentication".

[25]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[xx]		3GPP TS 33.222: " Generic Authentication Architecture (GAA); Access to network application functions using HypertextTransfer Protocol over Transport Layer Security (HTTPS)".

[yy]	IETF RFC 2616 (1999): "Hypertext Transfer Protocol (HTTP) – HTTP/1.1".

**** NEXT CHANGE ****

- 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| Solution #1: DNS request protection |  |  |  |  |  |  |  |  | x |  |
| Solution #2: Authentication between EEC and ECS based on primary authentication |  | x |  |  |  |  |  |  |  |  |
| Solution #3: Authentication/Authorization framework for Edge Enabler Client and Servers | x | x |  |  |  |  |  |  |  |  |
| Solution #4: Authentication/Authorization framework for Edge Enabler Client and Servers | x | x |  | x |  | x |  |  |  |  |
| Solution #5: Authentication and Authorization between the Edge Enabler Client and the Edge Enabler Server | x |  |  |  |  |  |  |  |  |  |
| Solution #6: Authentication and Authorization between the Edge Enabler Client and the Edge Enabler Server | x |  |  |  |  |  |  |  |  |  |
| Solution #7: Authentication and Authorization with the Edge Data Network | x | x |  |  |  | x |  |  |  |  |
| Solution #8: Authentication between EEC and EES | x |  |  |  |  |  |  |  |  |  |
| Solution #9: Authentication and authorization between EEC and ECS based on AKMA |  | x |  |  |  |  |  |  |  |  |
| Solution #10: Authentication and Authorization between the Edge Enabler Client and the Edge Configuration Server |  | x |  |  |  |  |  |  |  |  |
| Solution #11: Authentication between EEC and ECS |  | x |  |  |  |  |  |  |  |  |
| Solution #12: Onboarding and authentication/authorization framework for Edge Enabler Server and Edge Configuration Server |  |  | x |  |  |  |  |  |  |  |
| Solution #13: Transport security for EDGE-1-9 interfaces |  |  |  |  |  | x |  |  |  |  |
| Solution #14: Protection of Network Information Provisioning to Local AF directly |  |  |  |  |  |  | x |  |  |  |
| Solution #15: Network capability re-exposure via Edge Enabler Server |  |  |  |  |  |  |  | x |  |  |
| Solution #16: EEC authentication and authorization framework with ECS and EES | x | x |  |  |  |  |  |  |  |  |
| Solution #17: EEC/EES/ECS authentication and transport protection with TLS and HTTP Digest with AKMA PSK | x | x | x |  |  | x |  |  |  |  |
| Solution #18: Authentication and Authorization Framework for EDGE-4 interfaces using Primary authentication and proxy interface |  | x |  |  |  |  |  |  |  |  |
| Solution #19: Authentication/authorization between UE and Edge Data Network based on the secondary authentication |  |  |  | x |  |  |  |  |  |  |
| Solution #20: Authentication and authorization in EES capability exposure based on CAPIF |  |  |  |  |  |  |  | x |  |  |
| Solution #21: security for the interface between the SMF and LDNSR |  |  |  |  |  |  |  |  | x |  |
| Solution #22: EC: New solution on authorization during Edge Data Network change |  |  |  |  |  |  |  |  |  | x |
| Solution #X: Using TLS with AKMA to protect edge interfaces |  |  |  |  |  | X |  |  |  |  |
| #X: <Solution name> | X |  |  |  |  |  |  |  |  |  |

Editor’s Note: This clause provides the mapping of Solutions to Key Issues.

**** NEXT CHANGE ****

## 6.X	Solution #X: Using TLS with AKMA to protect edge interfaces

### 6.X.1	Solution overview

This solution addresses the EDGE-1 and EDGE-4 parts of key issue #6.

### 6.X.2	Solution details

#### 6.X.2.1	General

The solution proposes to use TLS with AKMA generated keys to protect EDGE-1 and EDGE-4. Profile of the Ua protocols for GBA (TS 33.220 [8]) are provided in the following clauses to achieve that protection. Any such standardisation of this profile would benefit from being in a AKMA related specification as they could apply to multiple use cases.

NOTE 1: It is not proposed to update the below profile in this specification if there is a specific work item for that work.

The solution proposed differs from solution #16 in not requiring an exchange of messages between EEC and ECS/EES to agree on the shared AKMA generated key before creating the TLS session.

NOTE 2: EEC authentication is not in the scope of this solution.

#### 6.X.2.2	Shared key-based UE authentication with certificate-based AF authentication

##### 6.X.2.2.1	General

The clause provides details needed to define solution similar to the Ua protocol given in 3GPP TS 33.222 [xx] rather than providing changes to Ua protocol given in TS 33.222 [xx].

6.X.1.2.2	Procedures

The procedures are similar to those given in clause 5.3.0 of TS 33.222 [xx] with the AKMA AF taking the role of the NAF from GBA (see TS 33.220 [8]), with the following changes.

At step 2, the client adds the constant string "3gpp-akma" to the "User-Agent" HTTP header as product tokens as specified in IETF RFC 2616 [yy].

At step 3, if the AF selects AKMA for deriving the key, then the AF includes the "3GPP-bootstrapping-akma" within the WWW-Authenticate header field.

At step 5 given AKMA has been selected for keying, the client sends a response with an Authorization header field where Digest is inserted using the A-KID as username. KAF is used as password in the Digest calculation.

At step 6 given AKMA has been selected for keying, the AF verifies the value of the password attribute using KAF retrieved from AAnF using the A-KID received as username attribute in the query. If the AF is not able to obtain the AF-specific key when using AKMA mode, the AF responds with an appropriate error message not containing the realm attributes from step 3.

#### 6.X.2.3	Shared key-based mutual authentication between UE and AF

##### 6.X.2.3.1	General

The clause provides details needed to define solution similar to the Ua protocol given in 3GPP TS 33.222 [xx] rather than providing changes to Ua protocol given in TS 33.222 [xx].

6.X.2.3.2	Procedures

The procedures are similar to those given in clause 5.4.0 of TS 33.222 [xx] with the AKMA AF taking the role of the NAF from GBA (see TS 33.220 [8]), with the following changes.

At step 2, the AF includes a constant string "3GPP-AKMA" as a PSK-identity hint to indicate that AKMA based keying is supported.

At step 3, the UE may use an AKMA generated key if suport was indiacted by the AF. To use as AKMA generated key, the UE derives the TLS premaster secret from KAF and sends a ClientKeyExchange message including a PSK identity consisting of "3GPP-AKMA" and the A-KID.

At step 4, if the AF receives the "3GPP-AKMA" prefix and the A-KID in the ClientKeyExchange messages it fetches the AF specific shared secret (KAF) from the AAnF using the A-KID. The AF derives the TLS premaster secret from the AF specific key (KAF).

### 6.X.3	Solution evaluation

TBD

**** END OF CHANGES ****