3GPP TSG-SA3 Meeting #102bis-e 	draft_S3-211063-r1

- e-meeting, 1 - 5 March 2021																    Revision of S3-21xxxx

- Source:	Qualcomm Incorporated

- Title:	Authorization of remote UE in L3 U2N relay

- Document for:	Approval

Agenda Item:	2.9

# 1	Decision/action requested

This contribution proposes a solution for authorization of remote UE for U2N relay.

# 2	References

[1]	TR 33.847 v0.4.0

# 3	Rationale

This solution addresses the KI #4.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

## 6.Y	Solution #Y: Authorization of the remote UE in L3 U2N relay

### 6.Y.1	Introduction

This solution addresses KI #4. This solution provides a mechanism to authorize a remote UE’s access to (1) a specific network slice(s) that requires slice-specific authentication and (2) a DN that requires a secondary authentication.

### 6.Y.2	Solution details

This solution reuses the existing slice-specific authentication and secondary authentication procedures specified in TS 33.501 [14].

To access a DN that requires a secondary authentication, the remote UE establishes an IPsec connection with the N3IWF via a L3 U2N relay if it has not been established yet, and then requests a PDU session via the N3IWF. The secondary authentication procedure is performed as specified in TS 33.501 [14].

To access a network slice that requires a slice-specific authentication, the remote UE establishes an IPsec connection with the N3IWF via a L3 U2N relay if it has not been established yet, and then sends a Registration Request or a Service Request to access the slice. The slice-specific authentication procedure is performed as specified in TS 33.501 [14].

Editor’s Note: Details on how the Remote UE is authorized to access a PDU Session/Slice to reach N3IWF to perform secondary A&A or NSSAA are FFS.

## 6.Y.3	Solution Evaluation

TBD.

***** END OF SECOND CHANGES *****