3GPP TSG-SA3 Meeting #102bis-e 	draft_S3-211070-r1

- e-meeting, 1 - 5 March 2021																    Revision of S3-21xxxx

- Source:	Qualcomm Incorporated

- Title:	5G MBS traffic protection at service-layer

- Document for:	Approval

Agenda Item:	2.11

# 1	Decision/action requested

This contribution proposes a new solution for 5G MBS traffic protection at the service-layer.

# 2	References

[1]	TR 33.850 v0.4.0

# 3	Rationale

This contribution proposes a new solution for 5G MBS traffic protection at the service-layer.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF FIRST CHANGES *****

## 6.x	Solution #x: Protection of MBS traffic at service layer based on GBA

### 6.x.1	Solution overview

This solution addresses Key Issue 2 and 3 to protect the MBS key and traffic at service-layer. This solution leverages the MBMS security architecture specified in TS 33.246 [3].

### 6.x.2	Solution details

In order to receive an MBS service, the UE establishes a secure connection with the MBS service function and obtains security materials

Figure 6.x.2-1: Message flows for MBS key delivery and MBS traffic protection

0. The UE is registered to 5GS.

1. The UE requests a PDU session establishment or modification to receive an MBS service.

2. The UE establishes a secure connection with MBSF-U based on GBA similar to MBMS [3] or AKMA [5]. In both scenarios, MBSF-U is considered an AF and UE and MBSF-U communicate using Ua/Ua* protocol. Both the UE and MBSF-U derive Multicast User Key (MUK) from the AF key (e.g., Ks_(int/ext)_NAF for GBA or KAF for AKMA).

Editor’s Note: the details of authentication protocol between UE and MBSF-U is ffs.

Editor’s Note: It is FFS whether the key management function needs to be separated from MBSF-U or not.

3. The UE receives the Multicast Service Key (MSK) from the MBSF-U. The MSK is protected using the MUK and delivered using a unicast message over the secure connection.

4. The UE receives the Multicast Traffic Key (MTK) protected using MSK from the MBSF-U. The MTK can be delivered either a unicast or a multicast message. The MTK is used as a root key to derive application/protocol specific keys to protect (e.g., encrypt or integrity protect) MBS service traffic.

5. Using the MTK received in step 4, the UE derives application/protocol specific keys and decrypts or verifies the MBS traffic.

Editor’s Note: more details on the key hierarchy and rekeying is FFS.

### 6.x.3	Solution evaluation

TBD

***** END OF FIRST CHANGES *****