3GPP TSG-SA3 Meeting #102Bis-e 	draft_S3-211115-r3

- e-meeting, 1 - 5 March 2021

- Source:	Ericsson

- Title:	UAS: Update to solution #2

- Document for:	Approval

Agenda Item:	2.7

# 1	Decision/action requested

Approve this contribution to update solution #2 in TR 33.854

# 2	References

[1] 3GPP TR 33.854 : "Study on security aspects of Unmanned Aerial Systems (UAS)"

[2] 3GPP TR 23.754 : " Study on supporting Unmanned Aerial Systems (UAS) connectivity, Identification and tracking"

# 3	Rationale

This contribution evaluates solution #2 in TR 33.854 [1] and solves the following ENs in the solution.

- Editor’s Note: Details of what information and how it is provided by MNO during the authentication and authorization procedure between UAV/UAVC and USS/UTM are FFS.

by adding the details in the procedure step, which are concluded in TR 23.754 [2] as follows.

During the UAV authentication/authorization procedure, the 3GPP system provides 3GPP identities/information for the UAV including the GPSI, optionally the PEI, the CAA-Level UAV ID and any aviation level information the UAV has provided to the 3GPP system. The USS/UTM uses such information for the authentication/authorization and for future interactions with the 3GPP system regarding the UAV via NEF/SCEF.

- Editor’s Note: The details of secure message exchanges between UE and USS/UTM in order to support A&A by USS/UTM (e.g. EAP or other framework/mechanism) is FFS.

by adding a text in step 6 to detail how secure message exchanges are executed.

- Editor’s Note: It is FFS whether USS/UTM is able to inform 3GPP about the action to be taken.

by removing the EN. Note that the followings are concluded in TR 23.754 [2]:

The 3GPP network shall be informed by the USS/UTM of the UAV authentication and authorization result and enforce the result accordingly.

Upon successful authorization, the USS/UTM notifies the UFES on the authentication/authorization result, including security info to be used by the UAV to set up a secure connection to the USS/UTM using the PDU session.

# 4	Detailed proposal

It is proposed to implement the following updates to solution #2 in TR 33.854.

*** START OF FIRST CHANGE ***

#### 6.2.2.1	Procedure for 5GS

Figure 6.2.2.1-1: UAV Authentication and Authorization procedure for 5GS

1.   Primary authentication is performed.

2.	A PDU session is established for the UE’s A&A request. The connection is allowed only between UAV/UAVC and UAS AF.

NOTE 1: The default policy for the PDU session on activation is to block any traffic from the UE except to the UAS AF.

NOTE 2: UAV/UAVC may want to connect to a DNN other than USS/UTM for some needs such as software updates. These type of PDU session request are out of this solution’s scope.

3.	UAV/ UAVC sends the request for authentication and authorization to the UAS AF over the user plane, e.g. including UAV/UAVC identity, USS/UTM identity (if available), and application level information, transparent to 3GPP and out of 3GPP scope, to be used by the USS/UTM when authenticating the UAV .

4.	The UAS AF gets the relevant subscription information from PCF or UDM with support from existing BSF functionality via SBA interfaces.

5.	UAS AF checks if the UAV has a valid aerial subscription based on the subscription information received from PCF or UDM. The UAS-AF learns the 3GPP UAV ID/GPSI from the BSF lookup and adds it to the CAA-Level UAV-ID and application level information that is forwarded to the USS/UTM.

NOTE 3:  Correlation of the 3GPP UAV ID and CAA-Level UAV-ID is performed by the USS/UTM.

If the check is successful, the UAS AF determines the USS/UTM serving the UAV/UAVC based on the USS/UTM identity provided in the request in Step 3 and the predefined list stored in UAS AF with valid USS/UTM identities including URLs to corresponding requests. If the requested identity is not in the list, the request from the UAV will be rejected. Otherwise, UAS AF sends A&A request towards the UTM/USS. The UAS AF provides 3GPP identities/information for the UAV including the GPSI and the CAA-Level UAV ID and any application level information the UAV has provided to the 3GPP system to the USS/UTM needed for further interaction between USS/UTM and 5GS regarding the PDU session. The request can contain an indication about the used mobile operator and 3GPP UAV/UAVC identity. Additionally, it forwards also the UAV/UAVC specific information received in the UAS A&A request. For the communication between UAS AF and USS/UTM, the NEF or the Common API framework (CAPIF) is used.

6.	An authentication and authorization procedure is executed between UAV/UAVC and USS/UTM. UAS AF relays the messages in this A&A procedure providing API’s, which use well-known web security mechanisms such as HTTPS, for the UAV-application and USS/UTM. USS/UTM considers the combined information from the UAV/UAVC and from the mobile network operator of the UAV/UAVC while performing the procedure. Note that if the information sent to the USS/UTM before this step is enough for the authentication&authorization procedure, there may be no need for extra message exchange between the UAV/UAVC and USS/UTM.

NOTE 4: The credentials and the method used in the UAS AA are out of 3GPP scope.

7.	USS/UTM sends UAS A&A result to UAS AF.

If the A&A result is successful, then USS/UTM may provide application specific information to be used for secure communication establishment between UAV and UTM/USS. This information is transparent to the network and the content is out of 3GPP scope. The network relays this information from the UTM/USS to the UAV.

If the A&A is unsuccessful, USS/UTM may inform the UAS AF about the action to take e.g. whether the PDU session established in Step 2 will be terminated.

8.	The UAS AF sends the response to the UAV.

9.	If the result of the A&A in Step 6 is successful, the UAS AF informs the SMF to modify the PDU session established in Step 2 such that the UAV/UAVC can communicate to the USS/UTM.

If A&A is not successful in Step 6, the UAS AF may inform the SMF to terminate the PDU session established in Step 2 according to the response from USS/UTM in Step 7.

NOTE 5: This solution does not enable/support authorization of UAV and UAVC pairing.

*** END OF FIRST CHANGE ***

*** START OF SECOND CHANGE ***

### 6.2.3	Solution evaluation

This solution fully addresses key issue #1 having no effect on the current NAS procedures and UAV UE proposing a UP solution.

Editor’s Note: Further evaluation is FFS whether mutual authentication is supported and whether this solution addresses 4th requirement (prevention of unauthorized USS)”.

This solution provides a user plane solution while TR 23.754 [3] concludes on using control plane based UUAA procedures.

This solution requires a new function (UAS AF) that requires updates on AMF and PCF for the subscription information requests.

It uses web-based interface for the communication between UAS AF and USS/ UTM.

Editor’s Note: The impacts related to the web-based interface is FFS.

*** END OF SECOND CHANGE ***