3GPP TSG-SA3 Meeting #102Bis-e	S3-211143-r1

- e-meeting, 1st – 5th Mar 2021

- Source:	Samsung

- Title:	Removal of Editor’s Note in Solution #5

- Document for:	Approval

Agenda Item:	2.17

# 1	Decision/action requested

It is requested to approve this contribution to update Solution #5 in TR 33.864.

# 2	Rationale

This contribution proposes to update Solution #5 about AMF re-allocation by re-directing UE to new AMF. Following Editor Notes are resolved:

- The solution behavior for various cases mentioned in clause 4.3 “Architecture and security assumptions” is FFS.

  - Added clause 6.5.2.1 to address this.

# 3	Detailed proposal

It is proposed to add following changes to Solution #5:

- 6.5	Solution #5: AMF re-allocation by re-directing UE to new AMF

- 6.5.1	Solution Overview

The solution proposes that the Initial-AMF, upon determining that AMF re-allocation is needed and it cannot communicate with the new Target-AMF directly, sends Registration Accept message to the UE, containing a re-route assistance information. The re-route assistance information includes following information:

-	A 5G-GUTI that is encoded for Target-AMF (set). It comprises of:

-	AMF-Set ID in GUAMI set to that of Target-AMF, as returned from NSSF/NRF

-	AMF Pointer set to 0xFFFFFF, or a reserved value

-	5G-TMSI set to random number

-	An indication that UE needs to re-register to the network using 5G-GUTI provided in re-route assistance information.

A UE, receiving this information in Registration Accept Message re-initiates registration procedure by sending Registration Request with 5G-GUTI.

RAN, upon receiving the new routing information (derived from 5G-GUTI) along with new Registration Request, directly forwards the request to an AMF in target AMF set.

The new (target) AMF, upon receiving registration request containing a 5G-GUTI whose AMF Pointer is set to a reserved value (or 0xFFFFFF), understands that this is a re-routed registration request and proceeds with Primary Authentication procedure. Thus, it can now proceed with setting up fresh security context and registration procedure can succeed.

- ****First Change***

- 6.5.2	Solution Details

Figure  6.5.2-1 shows the detailed procedure.

Figure 6.5.2-1: Redirecting UE to Target-AMF

Editor’s Note:	An update of the figure is needed to match the description.

Step #1:	UE initiates Registration procedure to connect to the network by sending registration request. RAN forwards the request to Initial-AMF. Initial-AMF may perform Primary authentication procedure.

Step #2:	Initial-AMF may perofrms NAS SMC procedure with the UE. From this point onwards, UE only accepts ciphered/protected messages from the network. As part of “Security Mode Complete” message, UE also sends complete Registration Request to the UE, which includes Requested S-NSSAIs.

Step #3:	Initial-AMF then initiates Nudm_SDM_Get procedure with UDM to download UE’s subscription data. The subscription data includes information about UE’s Subscribed S-NSSAIs.

Step #4:	Based on UE’s Requested S-NSSAIs, Subscribed S-NSSAIs and other (e.g. locally configured) information, Initial-AMF determines if it cannot serve all the S-NSSAI(s) from the Requested NSSAI permitted by the subscription information. Following sequence of events follows:

-	Initial-AMF invokes the Nnssf_NSSelection_Get service operation towards NSSF to retrieve Allowed NSSAI. The request to NSSF includes UE’s Requested S-NSSAIs, Subscribed S-NSSAIs, current tracking area (TAI) etc.

-	NSSF responds to Initial-AMF with an AMF-Set-ID, or a list of AMF nf-Instance-IDs (e.g. of Target-AMF) which are better suited to serve the UE, along with allowed NSSAI.

Editor’s Note:	How the solution works when NRF/NSSF returns a subset of AMFs is FFS.

Step #5:	If Initial-AMF, based on local configuration, determines that it cannot forward the Registration Request to Target-AMF directly and/or may need to go via RAN, it sends a Registration Accept message to the UE. The message includes re-route assistance information containing:

-	A 5G-GUTI that is encoded for Target-AMF (set). It comprises of:

-	AMF-Set ID in GUAMI set to that of Target-AMF, as returned from NSSF/NRF

-	AMF Pointer set to 0xFFFFFF, or a reserved value

-	5G-TMSI set to random number

-	An indication that UE needs to re-register to the network using 5G-GUTI provided in re-route assistance information.

Editor’s Note:	How the initial AMF learns about the new UE capability (to receive re-route assistance information) is FFS.

Step #6:	UE re-initiates Registration procedure by sending Registration Request with 5G-GUTI as received above. RAN forwards the request to Target-AMF.

Step #7:	Target-AMF, based on the presence of reserved value of AMF-Pointer in 5G-GUTI, determines that this is a re-routed Registration requests, and proceeds with identity request/response followed by Primary authentication procedure. Since no Inter-AMF routing via RAN is involved now, the registration procedure is able to proceed.

Editor’s Note:	How can UE process unprotected message from target AMF is FFS.

#### 6.5.2.1	Handling Different cases of communicating AMFs (Figure 4.3-1)

Case 1:		This is handled with procedure defined in Clause 6.5.1

Case 2.a.i:		This case cannot be handled with this solution, as source AMF does not transfer any data to target AMF.

Case 2.a.ii:	This case cannot be handled by any procedure.

Case 2.b.i:		It is proposed to handle this case as follows.

- UE does not discard 5G-GUTI pointing to old-AMF in Step #5, and sends it as an additional GUTI in the Registration Request in Step #6. Target-AMF, based on the presence of reserved value of AMF-Pointer in 5G-GUTI, determines that this is a re-routed Registration requests, and additional GUTI to be used to retrive the UE’s Context.

- Target-AMF requests old-AMF to transfer UE’s Context after UE has been successfully authenticated by Target-AMF. This is done by setting reason attribute to “MOBI_REG_UE_VALIDATED” as specified in clause 5.2.2.2.1.2 of 3GPP TS 29.518

Legacy UEs (that do not support receiving/sending new IEs) cannot handle this case.

Case 2.b.ii: 	Same as above.

6.5.3	 Evaluation

Pros:

- Since the solution does not involve routing security context via RAN, or introducing a new NF connecting isolated slices, there is no compromise with security.

Cons:

- Cannot handle Case 2.a.i.

Legacy UEs cannot handle Case 2.b.i, 2.b.ii.

NOTE:	This solution has impacts to SA2 procedures (e.g. as defined in 3GPP TS 23.502, Clause 4.2.2.2.3), and SA2 may need to be requested to review and update their specifications.

****End of Changes ***