3GPP TSG-SA3 Meeting #102Bis-e 	draft_S3-211147-r4

- e-meeting, 1 - 5 March 2021										Revision of S3-211147and S3-210846

- Source:	Thales, Nokia, Nokia Shanghai Bell

- Title:	TR 33.846: Evaluation of solution #2.2

- Document for:	Approval

Agenda Item:	2.5

# 1	Decision/action requested

Evaluation of solution #2.2.

# 2	References

[1]	3GPP TR 33.846 v0.10.0

# 3	Rationale

This contribution aims to address complete the evaluation of solution #2.2.

The evalution should keep track of the clarification given at the end of step 2 in clause 6.2.2.2: The authentication failure message for the first UE registration is left unencrypted, i.e.  the initial value of KAUSF is set to zero.

| Scenario | Topic | Issue with solution |
| --- | --- | --- |
| Case 1 | SUPI included in Authentication response | Issue 1: SUPI was never included before and it is newly proposed to include SUPI in Authentication response message. SUPI is never sent over the air in clear or in a format that is easy for the attacker to retrieve SUPI (Sent always only with concealment using ECIES as SUCI). But with this proposal, SUPI is concealed and sent using new encryption key KE, which makes the attacker easier to get the SUPI even though it is encrypted. Reason: KE is encryption key which is proposed in the solution and calculated from KAUSF, RAND, length of RAND, fixed string and length of a fixed string (KE=KDF (KAUSF, RAND ¦¦ length of RAND‖"Encryption Key"‖ Length of "Encryption Key")). KAUSF is zero for first UE Authentication failure message as described in solution. If KAUSF is zero, RAND is known value (in clear) received along with AUTN, rest of the parameters (not secret information) proposed is also known to attacker. With all these known parameters, KE can be generated by attacker or anyone. SUPI can be easily retrieved from the Authentication response with generated KE. |

# 4	Detailed proposal

It is proposed that SA3 review and approve the following pseudo-CR to 3GPP TR 33.846 [1].

*** START of CHANGE ***

#### 6.2.2.3	Evaluation

Editor's Note: Further evaluation is needed.

The protection of SQN will only work over 5G network.

- The solution has impact on the serving network when 5G AKA is used.

The solution requires changes to the serving network for 5G AKA.

FAIL CAUSE encryption: the authentication failure message for the first UE registration is left unencrypted, i.e.  the initial value of KAUSF is set to zero.

SUPI could lead to linkability attacks when SUPI is sent in the Authentication Response Message: Because a weaker encryption key KE is used to conceal SUPI, it could make it easier for the attacker to get the SUPI by cryptoanalysis. This is, because KE can be generated by anyone, if KAUSF is zero, since RAND is known value (in clear) received along with AUTN and the rest of the parameters is no secret information. If SUPI (optional field) is not included as part of Authentication response, then it doesn’t lead to linkability attack.

*** END of CHANGE ***