3GPP TSG-SA3 Meeting #102-e 	draft_S3-211166-r1

- e-meeting, 1 – 5 March 2021											Revision of S3-21xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	KI on SCP deployment models

- Document for:	Approval

Agenda Item:	5.20

# 1	Decision/action requested

KI added according to objectives of the SID added.

# 2	References

[1]	3GPP TS

# 3	Rationale

KI added according to objectives, i.e. copy from SID justification of the objective.

# 4	Detailed proposal

*********  START OF CHANGES

## 5.2	Key issue #2: SCP security domains

### 5.2.1	Key issue details

Editor’s note: SCP security domains to be defined.

TS 23.501 [X] addresses the aspects of handling multiple SCPs in indirect communication without and with delegated discovery and introduced SCP domains, which comprises multiple SCPs. NF Service Consumers or/and SCPs need to request NRF to discover the next hop SCP to route a service request from the NF Service Consumer to a NF Service Producer via multiple SCPs. 23.502 describes in the SCP profile SCP domain registration details about interconnected SCPs to and thus also identifies SCPs that interconnect domains.

The primary purpose of SCP domains is to describe the connectivity topology within a network. All SCPs within an SCP domain can directly interconnect. One SCP can be part of multiple SCP domains. In fact, the primary purpose of intermediate SCPs in the path is to interconnect SCP domains, thus, there are boundaries between SCP domains at each SCP in the path.

PLMN-wide trust between NFs and SCPs is an option, but more restrictions could be desirable in complex networks with SCP domains, e.g. if SCPs are operated in different regions/provinces. There can be several technical domains within a PLMN, where equipment with different capabilities is deployed and signalling also varies in some respects, e.g., if equipment upgrade is performed in a stepwise manner. Such technical domains can be defined based on computer centre boundaries, based on operators of subnetworks, based on regions/provinces, etc.

Figure 5.2.1-1: Illustration of SCP domains connecting via dedicated SPCs

This key issue is to study whether there is a need of one or several SCP domains becoming regions of trust of finer granularity than PLMN and whether there is a necessity of trust and policing of communication within or among such domains, i.e. for the case that request messages traverse a boundary between trust domains.

### 5.2.2	Security threats

In large PLMNs operated by different organizations in different regions/provinces or by slices operated by a tenant, PLMN wide trust between NFs and SCPs is possibly not wanted. If it is not possible to shield e.g. regions against each other, messages could traverse over several SCP domain boundaries, even though this has been denied.

5.2.3	Potential security requirements

### Editor's Note: ffs, maybe not applicable.

### 5.2.4	Architectural security requirement

5GS shall enable finer granular regions of trust than a PLMN, e.g. to shield regionally operated parts of the network against each other.

5GS shall be able to support security breaches to be locally contained.

Interconnection between trust regions shall be secured via SCPs.

*********  END OF CHANGES