# 3GPP TSG-SA3 Meeting #102-e 	S3-210484

# e-meeting, 18th - 29th January 2021	Revision of S3-20xxxx

# Source:	Qualcomm Incorporated

# Title:	Solution 19 EN resolution

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

Approve this contribution to resolve an EN in solution #19

# 2	References

[1]	3GPP TS 23.501: "System Architecture for the 5G System".

# 3	Rationale

The contribution resolves the following EN in solution 19.

- Editor’s Note: The protocol stack is FFS.

The control-plane and user-plane protocol stacks are added based on TS 23.501 [1]. The clause number and caption are updated accordingly. With such changes, the EN is deleted.

# 4	Detailed proposal

***	BEGINNING OF CHANGES ***

## 6.19	Solution #19: End-to-end security for the L3 UE-to-Network relay

### 6.19.1	Introduction

This solution addresses the KI #3. This solution provides a mechanism to provide an end-to-end security between a remote UE and the network for Layer 3 UE-to-network relay.

#### 6.19.2	Solution details

### 6.19.2.1 Procedure

Figure 6.19.2.1-1: Secure PC5 link establishment procedure for UE-to-network relay

1. The Remote UE establishes a secure PC5 link with the UE-to-network (U2N) relay.

2. The remote UE performs a registration procedure to 5GC via N3IWF as specified in clause 7.2.1 of TS 33.501 [14]. The only difference from the untrusted non-3GPP access procedure is that the UE-to-network relay and the serving network of the UE-to-network relay take the role of untrusted access network. As a result of successful registration via the N3IWF, IPsec tunnel is established between the remote UE and N3IWF and all traffic between the remote UE and N3IWF is end-to-end protected.

3. The remote UE may establish a PDU session via N3IWF for the traffic that requires end-to-end security between the remote UE and 5GC.

Editor’s Note: It is FFS how Remote UE connects to N3IWF after step 1.

#### 6.19.2.2 Protocol Stack

The protocol stacks for remote UE’s control-plane and user-plane via N3IWF are shown in Figure 6.19.2.2-1 and 6.19.2.2-2 respectively.

Figure 6.19.2.2-1 Control-plane protocol stack

Figure 6.19.2.2-2 User-plane protocol stack

***	END OF CHANGES ***