3GPP TSG-SA3 Meeting #103-e 	S3-212102

- e-meeting, 17th – 28th May 2021

- Source:	Samsung

- Title:	Updates to Evaluation in Solution #5

- Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

It is requested to approve this contribution to update Solution #5 in TR 33.864.

# 2	Rationale

pCR S3-210961 added a number of evaluation comments to Solution #5 in SA3 #102-e-Bis. This contribution proposes to provide clarification to some of these comments and update the evaluation accordingly. Also moved the NOTE to the end for better readability.

3. This solution removes the option of NAS reroute via RAN. How to solve the registration failure issue when RR is rerouted via RAN is still not solved.

This solution actually removes the need of NAS reroute via RAN, and hence we will no longer see the registration failure issue due to NAS reroute via RAN.

SA2 request to SA3 is to “resolve the security issue with NAS reroute”, and not to “resolve the security issue with NAS reroute in a specific way”.

4. This solution has impact on the UE.

Fine keeping this, however this is true for a number of solutions.

3	Detailed proposal

It is proposed to add following changes to Solution #5:

*** First Change ***

### 6.5.3	 Evaluation

-	Since the solution does not involve routing security context via RAN, or introducing a new NF connecting isolated slices, there is no compromise with security.

-	Cannot handle Case 2.a.i.

-	Legacy UEs cannot handle Case 2.b.i, 2.b.ii.

- The initial AMF is mandated to assign a GUTI to the UE on behalf of the target AMF.

- The target AMF is mandated to run identity request and perform primary authentication again.

The target AMF is unable to retrieve UE context from the old AMF for legacy UEs. Context loss is inevitable.

- This solution removes the option of NAS reroute via RAN and doesn’t conform to existing SA2 procedures.

- This solution has impact on the UE, though legacy UEs are handled as well.

- The UE and the network are required to run two registration procedures.

NOTE:	This solution has impacts to SA2 procedures (e.g. as defined in 3GPP TS 23.502, Clause 4.2.2.2.3), and SA2 may need to be requested to review and update their specifications.

*** End of Changes ***