3GPP TSG-SA3 Meeting #103-e 	S3-211558-r1

- e-meeting, 17 - 28 May 2021											Revision of S3-20xxxx

- Source:	ZTE

- Title:	Add evaluation and update comparison table for solution 3.2

- Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

Add evaluation and update comparison table for solution #3.2.

# 2	References

[1]	3GPP TR 33.846 v0.11.0

# 3	Rationale

Propose to add evaluation and update the comparison table for solution #3.2.

# 4	Detailed proposal

Changes to clause 6.3.2.3 and 7.0.2

**** START OF 1st CHANGE ****

#### 6.3.2.3	Evaluation

The solution mitigates the SUCI guessing attack by introducing a new parameter Check Value behind SUPI.

The solution requires changes on the USIM and the UDM.

The length of the Check Value depends on the network operator. The Check Value may be updated by the home network through the way update routing indicator in SUCI. There is an impact on the UICC and the UE Parameter Update (UPU) procedure if the network needs to support the update of the Check Value via UPU.

**** END OF 1st CHANGE ****

**** START OF 2nd CHANGE ****

### 7.0.2	Impact of solutions

Table: Comparison table of UE and network impacts per solution

|  | UE impacts | Network impacts |  | comments |
| --- | --- | --- | --- | --- |
| Solutions |  | AMF/ SEAF | UDM/ ARPF |  |
| Solutions for resilience against identifier linkability |  |  |  |  |
| #2.1: Handling of Sync failure by AUTS encryption | ME | X | X | Risk of linkability attack. |
| #2.2: Encryption of authentication failure message types by UE with new keys derived from K_AUSF | Probably ME | X | X | Authentication failure message for the first UE registration is left unencrypted |
| #2.3: Unified authentication response message by UE | USIM | X | X |  |
| #2.4: MAC-S based solution | USIM | X | X |  |
| #2.5: Encryption of authentication failure message with SUCI method | USIM or ME | X | X | This solution relies on the availability of SUCI mechanism. |
| #2.6: Certificate based encryption of unicast NAS message | USIM and ME | X | X | This solution relies on the availability of PKI architecture, and there is companion list of open questions. |
| #2.7: Mitigation against the SUCI replay attack | USIM and ME | X | X |  |
| #2.9: MAC, SYNCH failure cause concealment | USIM | X | X |  |
| Solutions for availability aspects of SUCI usage |  |  |  | Usage of the long term key K for other uses. |
| #3.1: Mitigation of SUPI guessing and SUCI replay attack using long term key | USIM and ME | X | X | Security issue due to the use of the long term key K for other purpose than AKA. |
| #3.2: Adding Check Value behind SUPI to mitigate the SUPI guessing attacks | USIM |  | X |  |
| Solutions on re-synchronisation in AKA |  |  |  |  |
| #4.1: Using MACS as freshness in the calculation of AK | USIM |  | X |  |
| #4.2: Using symmetric encryption function to protect SQN during a re-synchronisation procedure in AKA | USIM |  | X | This solution requires the definition of a new function f6* for AKA procedure |
| #4.3: SQN protection by concealment with SUPI in USIM | USIM | X | X | Solution works if SUCI computation is performed in the USIM. |
| #4.4: SQN protection during re-synchronisation procedure in AKA | USIM and ME |  | X | Limitation of SQNMS leakage depends on RAND storage in the USIM. |
| #4.5: AUTS SQNMS solution for 5GS | USIM and ME | X | X | TBD |
| #4.6: Using time-based or partly time-based SQN generation |  |  |  | Solution requires time based SQN generation |
| #4.7: SQN protection by concealment with SUPI with f5* | USIM and ME | X | X | TBD |

NOTE:	A mapping table on which solution is addressing which key issue is provided in clause 6.0.

**** END OF 2nd CHANGE ****