3GPP TSG-SA3 Meeting #103-e 	draft_S3-211974-r2

- e-meeting, 17 - 28 May 2021											Revision of S3-21XXXX

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Solution on Access token request for NF Set

- Document for:	Approval, Information, Discussion

Agenda Item:	5.20

# 1	Decision/action requested

Solution on authorization of multiple consumers within a NF set.

# 2	References

[1]	3GPP TS 33.875

# 3	Rationale

The usefulness of having reusage of the same access token within a NF set is shown by this solution.

# 4	Detailed proposal

*********** START OF CHANGES

6.X	Solution #X: Access token request for NF Set

Editor’s Note: It is ffs whether using the same access token for different OAuth 2.0 clients follows the OAuth 2.0 RFC and best practices.

### 6.X.1	Introduction

This solution addresses KI#Y.

SBA introduces the concepts of NF Set and NF Service Set. 5G SBA architecture design allows for the concept of stateless NFs. To avoid that a NF from a NF Set needs to request a new access token, when targeting a service of an existing resource requested before by another NF of the NF Set, it is proposed that any NF in a NF Set can request an access token for the NF Set. Thus, any NF Service Consumer targetting a service of an existing resource it can use the access token provided to a NF Set of NF Service Consumers.

### 6.X.2	Solution details

The NF Service Consumer belonging to a NF Set includes its NF Set ID in the Access Token Request message to NRF and also in the CCA or the NF certificate.

When the Access Token Request is processed by the NRF and a NF Set ID is included, the NRF knows that the NF Service Consumer requests an access token to be usuable by all NF Service Consumer instances within the NF Set. If NRF authorization of the NF Service Consumer is successful and, if CCA was provided by the NFc, the NF Set ID in the CCA matches the NF Set ID in the access token, otherwise it can match it against the NF Set ID in the NF certificate, NRF includes as claim the NF Set ID of the expected NF Service Consumer instances to allow the access token generated for usage by all NF Service Consumers in the NF Set. NRF sends the access token back to the requester.

Figure 1 – Access Token Request procedure (TS 33.501 Figure 13.4.1.1.1-1) enhanced with NF Set ID in the Access Token Request message

When a service is requested, the requester (NF Service Consumer or SCP) includes the NF Set ID of the NF Service Consumer in the Service API Request as well as in the CCA, if the CCA is sent, in addition to the access token obtained from the NRF. Alternatively, the NF Set ID can be included in the NF certificate.

The NF Service Producer checks whether the Consumer NF Set Id in the Service Request matches with the NF Set ID claim in the Access token. If CCA is sent, it also verifies, if the NF Set ID matches the NF Set ID I the CCA. If included in NF certificate, it can also match the NF Set ID with the NF Set ID in the NF certificate. If yes, it proceeds with serving the request, otherwise it rejects the request.

### 6.X.3	Evaluation

TBD

*********** END OF CHANGES