3GPP TSG-SA3 Meeting #103-e 	S3-212080-r1

- e-meeting, 17 - 28 May 2021											Revision of S3-20xxxx

- Source:	Samsung

- Title:	Solution on user's consent for exposure of information to Edge Applications

- Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is proposed to approve this pCR to add a solution on user consent for exposure of information to edge application to UC3S study TR 33.867.

# 2	References

[1] 3GPP TR 33.867: “Study on User Consent for 3GPP services (Release 17)”

[2] 3GPP TS 23.558: “Architecture for enabling Edge Applications (EA)”

# 3	Rationale

This contribution proposes a solution on user consent for exposure of information to Edge Applications, addressing key issue 6.1. This solution proposes a mechanism to obtain a grant of the user consent from an authorized user for providing the service to the edge application.

# 4	Detailed proposal

*****Start of Change*****

- 7.Y	Solution #Y: User's consent for exposure of information to the Edge Applications – Network Triggered

- 7.Y.1	Introduction

This solution addresses the security requirement on user's consent for exposure of information to Edge Applications in key issue #6.1

User's consent grant is obtained by application client based on OTP (One Time Password) generated by Edge enabler server.

- 7.Y.2	Solution details

Figure 7.Y.2-1: User's consent for exposure of information to the Edge Applications

NOTE 1: 	This solution is not impacting any 3GPP protocols/ procedures.

NOTE 2: 	Mechanism for the distribution of OTP is not restricted in this solution alternative. It may be a SMS, EMAIL, URL.

- The application client sends the request for the user consent to the EAS, when the application client needs the user consent.

- Upon receiving the request for the user consent from the application client, the EAS shares the request for the user consent to the EES.

- The EES generates the OTP for obtaining the user consent.

- As a part of registration, the application client provides the necessary user contact details to the EES. The EES shares the OTP with the UE. A channel of sharing the OTP by the EES to the UE is determined based on the user contact information such as a mobile number, email address, etc. available at the EES.

- Upon requesting the user consent with the EAS, the application client initiates steps to obtain the user consent grant from the user.

- The application client indicates the delivery of OTP and requests the user using the UE to input the OTP in the consent for providing the consent to the application client.

- The UE shares the OTP and the user consent given by the user with the application client.

- The application client notifies the user consent and the OTP to the EAS using the user consent grant notification.

- The EAS notifies the user consent and the OTP to the EES, in response to receiving the user consent grant notification from the EAS.

- Upon receiving the user consent and the OTP, the EES verifies the received OTP with the OTP generated.

- Upon matching both the OTPs, the EES responds with the user consent to the EAS.

- The EES stores the user consent.

- The EAS responds the user consent with the application client, in response to receiving the user consent.

NOTE 3: 	The validation of user consent and the frequency of consent is decided by EES and this solution does not concern itself which entity enforces the user consent.

Editor's Note: How consent is handled during EES mobility is FFS.

- Editor's Note: How the user changes the consent is FFS

*****End of Change*****