3GPP TSG-SA3 Meeting #103-e 	draft_S3-212115-r1

- e-meeting, 17 - 28 May 2021											Revision of S3-20xxxx

- Source:	Ericsson

- Title:	UP IP: Update to solution #21 (Interworking handover from EPS to 5GS)

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes an update to solution #21 (Interworking handover from EPS to 5GS) in [1].

# 2	References

[1]	3GPP TR 33.853 "Key issues and potential solutions for integrity protection of the user plane "

# 3	Rationale

In LS response from SA2 in S3-211398, SA2 indicates that they have selected solution #15 in [1] where the PGW-C+SMF provides the UP security policy to the target RAN via target AMF at Interworking handover from EPS to 5GS.

In intra-system NG HO, the UP IP policy is not included in Source to Target Transparent Container. Therefore the additional updates are proposed:

-UP IP policy is removed from Source to Target Transparent Container;

UP security policy is not included in UE Context Modification Request message in TS 38.314:

-UP IP policy is removed from UE Context Modification Request message.

It is also proposed to update the evaluation clause as this solution has been aligned with solution #15.

# 4	Detailed proposal

It is proposed to update solution #21 in the study in [1].

**** START OF CHANGES ****

## 6.21	Solution #21: Interworking handover from EPS to 5GS

### 6.21.1	Introduction

This solution addresses key issue #7: Ensuring UP IP is enforced at interworking.

The PGW-C+SMF provides the UP IP policy per PDU session and provides the policies to the target gNB/ng-eNB.

This solution impacts the UE, the MME, the AMF, the SMF, the eNB and the ng-eNB and gNB.

### 6.21.2	Network options affected

This solution is applicable to the following network options:

- Option 1 – eUTRA with EPC

- Option 2 – NR standalone with 5G Core

- Option 3 – EPC based Dual Connectivity of eUTRA and NR RAT

- Option 4 – 5G core based Dual Connectivity (NR master – eUTRA secondary)

- Option 5 – 5G core with eUTRA

- Option 7 – 5G core based Dual Connectivity (eUTRA master – NR secondary)

### 6.21.3	Solution description

"UE capability indicating UP IP support" in this clause and it’s sub-clauses refers to "UE capability indicating UP IP support over eUTRA when connected to 5GC”.

#### 6.21.3.1	Upgraded RAN nodes and core network nodes to support UP IP

The solution is to describe the behaviour of the source eNB node and the target ng-eNB node at interworking handover from EPS to 5GS when the source eNB and the target ng-eNB/gNB support UP IP.

The procedure described below applies to when UP IP policy is set to either: "required", "preferred" or "not needed".

##### 6.21.3.1.1 	Interworking handover from EPS to 5GS

The source eNB has received a UP IP policy from the MME per radio bearer established with the UE. The source eNB includes the UP IP activation status in the Source to Target Transparent Container to the target ng-eNB/gNB.

The target AMF/SMF shall ignore any UP IP policy received from the source MME.

The PGW-C+SMF provides a UP IP policy as described in TS 23.501. The SMF responds to the target AMF including a UP IP policy per PDU session to be handovered to 5GS which the AMF forwards to the target ng-eNB/gNB.

A target ng-eNB/gNB which supports UP IP is able to activate UP IP according to the received UP IP policy and the "UE capability indicating UP IP support over eUTRA when connected to 5GC” received from the source MME via target AMF.

NOTE: If the UP IP indication reuses 1 bit of EPS security capability, the protection of UE EPS security capability in interworking from EPS to 5GS can be reused for protection of indication of UE support UP IP in E-UTRA.

Figure 6.21.3.1.1: Interworking handover from EPS to 5GS

Step 1: The source eNB includes the UP IP activation status in the Source to Target Transparent Container  in Handover Required message to the source MME.

Step 2: The source MME forwards the Source to Target Transparent Container together with the "UE capability indicating UP IP support over eUTRA when connected to 5GC” to the target AMF.

Step 2b: The target AMF invokes the Nsmf_PDUSession_CreateSMContext Request service operation with the SMF. The target AMF/SMF shall ignore any UP IP policy received from the source MME. The SMF provides a UP IP policy per PDU session as described in TS 23.501.

Step 2c: The SMF responds with the Nsmf_PDUSession_CreateSMContext Response to the target AMF including a UP IP policy per PDU session to be handovered to 5GS.

Step 3: The target AMF forwards the Source to Target Transparent Container together with the "UE capability indicating UP IP support over eUTRA when connected to 5GC” and the UP IP policy per radio bearer to the target ng-eNB/gNB.

Step 4: The target ng-eNB/gNB prepares the Target to Source Transparent Container to the source eNB. Confirmation of valid "UE capability to support UP IP over E-UTRA when connected to 5GC" and valid UP IP activation status included in the Source to Target Transparent container is made at the target ng-eNB/gNB when the core network includs the stored "UE capability to support UP IP over E-UTRA when connected to 5GC" and the UP IP policy via the target AMF to the target ng-eNB/gNB. If the target ng-eNB/gNB becomes aware that there is a mismatch between the "UE capability to support UP IP over E-UTRA when connected to 5GC" and the UP IP activation status received from the source eNB in the Source to Target Transparent container and the ones stored and received from the core network, then the target ng-eNB/gNB takes the "UE capability to support UP IP over E-UTRA when connected to 5GC" and the UP IP policy stored and received from the core network into use.

The target ng-eNB/gNB activates UP IP for the radio bearers to be handovered to the target ng-eNB/gNB. The target ng-eNB/gNB sends the Handover Request Acknowledgement message including the Target to Source Transparent Container to the target MME.

Step 5: The target AMF forwards the parameters received in Handover Reqest Acknowledge message to the source MME in the Forward Relocation Response message.

Step 6: The source MME forwards the parameters received in Forward Relocation Response message to the source eNB in the Handover Command message.

Step 7: The source eNB initiates Handover Command procedure with the UE.

Step 8: The UE confirms by sending a Handover Complete message to the target eNB.

Step 9: The target ng-eNB/gNB notifies the target AMF that the handover procedure is completed by sending a Handover Notify message.

Step 10. The UE initiates a Registration procedure with the target AMF including the "UE capability indicating UP IP support over eUTRA when connected to 5GC" in the Registration Request message.

Step 11: If the target AMF identifies a mismatch with the previously stored UE capability and becomes aware that the target ng-eNB/gNB doesn’t know the "UE capability indicating UP IP support over eUTRA when connected to 5GC” after an interworking handover from EPS, the target AMF shall send the "UE capability indicating UP IP support over eUTRA when connected to 5GC" to the target ng-eNB/gNB in the UE Context Modification Request message.

Step 12: If the target ng-eNB identifies a mismatch between the stored capabilities and the received capabilities then the target ng-eNB may activate UP IP for the DRB(s) established with the UE.

Step 13: The target ng-eNB/gNB responds to the target AMF by sending a UE Context Modification Response message when the RRC Reconfiguration procedure is completed.

#### 6.21.3.2	Handling of legacy eNB and legacy ng-eNB

##### 6.21.3.2.1	Handling of legacy source eNB

A legacy source eNB node has not be upgraded to support UP IP, and cannot activate UP IP for radio bearers established with the UE and therefore cannot handover data radio bearers with UP IP activated to 5GS.

After the HO is completed the UE initiates a Registration procedure with the target AMF including the "UE capability indicating UP IP support over eUTRA when connected to 5GC". If the target AMF identifies a mismatch with the previously stored UE capabilities and becomes aware that the target ng-eNB/gNB doesn’t know the "UE capability indicating UP IP support over eUTRA when connected to 5GC" after an interworking handover from EPS, the AMF should send the "UE capability indicating UP IP support over eUTRA when connected to 5GC" to the target ng-eNB/gNB via the UE Context Modification Request message. The target ng-eNB/gNB may activate UP IP for the data radio bearer established with the UE and/or respond to the target MME by sending a UE Context Modification Response message.

##### 6.21.3.2.2	Handling of legacy target ng-eNB

A legacy target ng-eNB node has not been upgraded to support UP IP and cannot activate UP IP for a radio bearer handovered from the source eNB.

Editor’s note: If the UP IP policy is set to "required", it’s FFS how the source eNB identifies that the target ng-eNB is not supporting UP IP,

### 6.21.4	Solution evaluation

Editor's Note: write an evaluation of the solution against the relevant key issues here (or have an editor's note saying that it is to be added later.)

Confirmation of valid "UE capability to support UP IP over E-UTRA when connected to 5GC", valid UP IP activation status included in the Source to Target Transparent container at the target ng-eNB/gNB, is achieved by the core network including the stored "UE capability to support UP IP over E-UTRA when connected to 5GC", the UP IP policy via the target AMF to the target ng-eNB/gNB. If the target ng-eNB/gNB becomes aware that there is a mismatch between the "UE capability to support UP IP over E-UTRA when connected to 5GC" and the UP IP activation status received from the source eNB in the Source to Target Transparent Container and the ones stored and received from the core network, then the target ng-eNB/gNB takes the "UE capability to support UP IP over E-UTRA when connected to 5GC" and the UP IP policy stored and received from the core network into use.

**** END OF CHANGES ****