3GPP TSG-SA3 Meeting #101e 	S3-211903

# e-meeting, 9 – 20 November 2020													Revision of S3-20xxxx

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | DRAFT | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS SCAS: Adding the assets and threats of the new NFs and update existing threats |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_IMS |  |  |  |  |  | Date: |  |  | 2020.10.28 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add assets and threat description to TR 33.926 in support of the the IMS SCAS work, i.e. Media related NFs (MRFP, IMS MGW,MGCF), IMS-AGW, and TrGW. The asset of the above network funtions should be captured. For the IMS signalling transport threats, it assumes that the UE shall always support non-null encryption algorithm, which is not realistic during the IMS deployment. On the other hand, the most reasonable test case on data encryption is also covered by the last bullet. Hence, it is proposed to remove the threats on the case that P-CSCF may be configured to never apply confidentiality. It is unclear about how to implement the priority of SIP transaction in MRFC, which is left to national regulation and network configuration. It is proposed to remove it either. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add assets and threats related to Media related NFs (MRFP, IMS MGW,MGCF), IMS-AGW, and TrGW. Two threats in P-CSCF and MRFC are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No reference of critical assets and threats for Media related NFs (MRFP, IMS MGW,MGCF), IMS-AGW, and TrGW. Two threats are not reasonable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st changes ****************

X.a	Assets and threats specific to the IMS AGW

X.a.1	Critical assets

In addition to the critical assets of a GNP has been described in clause 5.2 of the present document, the critical assets specific to the IMS AGW to be protected are:

-	IMS AGW Application;

-	Media stream resource;

-	Security data, i.e. cryptographic materials for Iq and Mp interfaces

-	The interfaces of the IMS AGW to be protected and which are within SECAM scope:

-	Iq interface between the IMS AGW and P-CSCF

-	Mb interface between the IMS AGW and IMS MGW

-	Console interface, for local access: local interface on the IMS AGW

-	OAM interface, for remote access: interface between the IMS AGW and the OAM system

NOTE 1: 	The detailed interfaces of the IMS AGW class are described in clause 4 of the present document.

-	IMS AGW Software: binary code or executable code

NOTE 2: 	IMS AGW files may be any file owned by a user (root user as well as non-root users), including user account data and credentials, log data, configuration data, OS files, IMS AGW application, user plane security mechanism, or cryptographic materials.

X.b	Assets and threats specific to the MRFP

X.b.1	Critical assets

In addition to the critical assets of a GNP has been described in clause 5.2 of the present document, the critical assets specific to the MRFP to be protected are:

-	MRFP Application

-	Media stream resource

-	Security data, i.e. cryptographic materials for Mp interface

-	The interfaces of the MRFP to be protected and which are within SECAM scope:

-	Mp interface between the MRFC and MRFP

-	Console interface, for local access: local interface on the MRFP

-	OAM interface, for remote access: interface between the MRFP and the OAM system

NOTE 1: 	The detailed interfaces of the MRFP class are described in clause 4 of the present document.

-	MRFP Software: binary code or executable code

NOTE 2: 	MRFP files may be any file owned by a user (root user as well as non-root users), including user account data and credentials, log data, configuration data, OS files, MRFP application, user plane security mechanism, or cryptographic materials.

X.c	Assets and threats specific to the IMS MGW

X.c.1	Critical assets

In addition to the critical assets of a GNP has been described in clause 5.2 of the present document, the critical assets specific to the IMS MGW to be protected are:

-	IMS MGW Application;

-	Media stream resource;

-	Security data, i.e. cryptographic materials for Mn, Mb, and CS interfaces

-	The interfaces of the IMS MGW to be protected and which are within SECAM scope:

-	Mn interface between the IMS MGW and MGCF

-	Mb interface between the IMS MGW and MRFP/IMS AGW

-	CS interface between the IMS MGW and CS Network

-	Console interface, for local access: local interface on the IMS AGW

-	OAM interface, for remote access: interface between the IMS MGW and the OAM system

NOTE 1: 	The detailed interfaces of the IMS MGW class are described in clause 4 of the present document.

-	IMS MGW Software: binary code or executable code

NOTE 2: 	IMS MGW files may be any file owned by a user (root user as well as non-root users), including user account data and credentials, log data, configuration data, OS files, IMS MGW application, user plane security mechanism, or cryptographic materials.

X.d	Assets and threats specific to the TrGW

X.d.1	Critical assets

In addition to the critical assets of a GNP has been described in clause 5.2 of the present document, the critical assets specific to the TrGW to be protected are:

-	TrGW Application;

-	Media stream resource;

-	Security data, i.e. cryptographic materials for Ix and Izi interfaces

-	The interfaces of the TrGW to be protected and which are within SECAM scope:

-	Ix interface between the TrGW and IBCF

-	Izi interface between the TrGW and IP Multimedia Network

-	Console interface, for local access: local interface on the TrGW

-	OAM interface, for remote access: interface between the TrGW and the OAM system

NOTE 1: 	The detailed interfaces of the TrGW class are described in clause 4 of the present document.

-	TrGW Software: binary code or executable code

NOTE 2: 	TrGW files may be any file owned by a user (root user as well as non-root users), including user account data and credentials, log data, configuration data, OS files, TrGW application, user plane security mechanism, or cryptographic materials.

X.e	Assets and threats specific to the MGCF

X.e.1	Critical assets

In addition to the critical assets of a GNP has been described in clause 5.2 of the present document, the critical assets specific to the IMS AGW to be protected are:

-	MGCF Application;

-	Media stream resource;

-	Security data, i.e. cryptographic materials for Mg, Mj, CS, and Mn interfaces

-	The interfaces of the MGCF to be protected and which are within SECAM scope:

-	Mg interface between the MGCF and I-CSCF

-	Mj interface between the MGCF and BGCF

-	CS interface between the MGCF and CS Network

-	Mn interface between the MGCF and IM MGW

-	Console interface, for local access: local interface on the IMS AGW

-	OAM interface, for remote access: interface between the IMS AGW and the OAM system

NOTE 1: 	The detailed interfaces of the IMS AGW class are described in clause 4 of the present document.

-	IMS AGW Software: binary code or executable code

NOTE 2: 	IMS AGW files may be any file owned by a user (root user as well as non-root users), including user account data and credentials, log data, configuration data, OS files, IMS AGW application, user plane security mechanism, or cryptographic materials.

*************** End of the 1st changes ****************

*************** Start of the 2nd changes ****************

X.2.3	Threats related to IMS signalling transport

- Threat name:  No protection or weak protection for IMS signalling data.

- Threat Category: Tampering, Information Disclosure.

- Threat Description: The following behaviours may lead to bidding down attacks

-	If the protection implemented for the IMS signalling over Gm interface uses the wrong security profile, which may contain weak security algorithms or protocol versions known to be vulnerable, the level of the security of the IMS signalling data may be degraded and fail to fulfil the required security.

-	If the P-CSCF policy requires confidentiality, then all UEs with no encryption support would be denied access to the IMS network. For example, if the UE sends the NULL encryption algorithm to the P-CSCF in SM1, and the SM1 message is not denied by the P-CSCF, the following negotiated SA between UE and P-CSCF may be established without confidentiality protection, which disobeys the P-CSCF policy requiring confidentiality. Hence, the following IMS signalling data will be leaked.

- Threatened Asset: IMS signalling data.

*************** End of the 2nd changes ****************

*************** Start of the 3rd changes ****************

X.7	Assets and threats specific to the MRFC

X.7.1	Critical assets

In addition to the critical assets of a GNP has been described in clause 5.2 of the present document, the critical assets specific to the MRFC to be protected are:

-	MRFC Application

-	Media stream resource, Charging data records

-	Security data, i.e. cryptographic materials for Mp, Mr, and Cr/Mr’ interfaces

-	The interfaces of the MRFC to be protected and which are within SECAM scope:

-	Mp interface between the MRFC and MRFP

-	Mr interface between the MRFC and the S-CSCF

-	Cr/Mr’ interface between the MRFC and AS

-	Console interface, for local access: local interface on the MRFC

-	OAM interface, for remote access: interface between the MRFC and the OAM system

NOTE 1: 	The detailed interfaces of the MRFC class are described in clause 4 of the present document.

-	MRFC Software: binary code or executable code

NOTE 2: 	MRFC files may be any file owned by a user (root user as well as non-root users), including user account data and credentials, log data, configuration data, OS files, MRFC application, user plane security mechanism, or cryptographic materials.

*************** End of the 3rd changes ****************