3GPP TSG-SA3 Meeting #103-e 	S3-212332

# e-meeting, 17 - 28 May 2021											                    Revision of S3-211858

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.512 | CR | Draft CR | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New test case on validation of S-NSSAIs in PDU session establishment request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSCAS_5G |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | R-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | After the successful network slice-specific authentication and authorization, there will be an Allowed NSSAI list both in UE and AMF. Then, the UE will initiate the PDU session establishment request with the requested S-NSSAIs included. As defined in TS 24.501, clause 5.4.5.2.5, if the Request type IE is set to "initial request" and the S-NSSAI IE contains an S-NSSAI that is not allowed by the network, then the AMF shall send back to the UE the 5GSM message which was not forwarded. If the AMF does not perform the validation, an attacker can still include the rejected S-NSSAIs in the request and access the service of the slice after it fails the NSSAA procedure. Hence, new test case on this valication is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new test case on S-NSSAIs validation during PDU session establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the AMF does not perform the S-NSSAIs validation, an attacker can still include the rejected S-NSSAIs in the request and access the service of the slice after it fails the NSSAA procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*****************Start of the changes************************

4.2.2.x	Security in PDU session establishment procedure

4.2.2.x.1	Validation of S-NSSAIs in PDU session establishment request

Requirement Name: validation of S-NSSAIs in PDU session establishment request

Requirement Reference: TS 24.501 [5], clause 5.4.5.2.5

Requirement Description:"

13)	if the Request type IE is set to "initial request" and the S-NSSAI IE contains an S-NSSAI that is not allowed by the network, then the AMF shall send back to the UE the 5GSM message which was not forwarded as specified in subclause 5.4.5.3.1 case e) or case f);" as specified in TS 24.501 [5], clause 5.4.5.2.5.

Threat References: TR 33.926 [6], clause K.2.x, Incorrect Validation of S-NSSAIs

Test Case:

Test Name: TC_VALIDTATION_SNSSAI_IN_PDU_REQUEST

Purpose:

Verify that S-NSSAIs which are not within Allowed NSSAI list are not accepted by the AMF under test in PDU session establishment procedure.

Pre-Conditions:

Test environment with UE, UDM, SMF and NSSAAF, which may be simulated.

The tester configures UDM with an S-NSSAI that require Network Slice-Specific Authentication and Authorizationin in UE’s subscription information.

Execution Steps

A.	Test Case 1

- The UE sends the S-NSSAI that require NSSAA to the AMF under test using registration request message.

- After receiving the NSSAA request from the AMF, the NSSAAF sends EAP success to AMF.

- The UE sends PDU session establishment request to the AMF with the S-NSSAI.

B.	Test Case 2

- The UE sends the S-NSSAI that require NSSAA to the AMF under test using registration request message.

- After receiving the NSSAA request from the AMF, the NSSAAF sends EAP failure to AMF.

- The UE sends PDU session establishment request to the AMF with the S-NSSAI.

Expected Results:

For test case 1, the AMF continues the PDU session establishment procedure by sending a Nsmf_PDUSession_CreateSMContext Request to the SMF.

For test case 2, the AMF aborts the PDU session establishment procedure by sending back the 5GSM message to the UE.

Expected format of evidence

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

*****************End of the changes************************