3GPP TSG-SA3 Meeting #104-e ad-hoc 	S3- 213310-r2

- e-meeting, 27 – 30 September 2021	Revision of S3-21xxxx

- Source:	InterDigital

- Title:	TR 33.847 solution #25 evaluation

- Document for:	Approval

Agenda Item:	5.2

# 1	Decision/action requested

It is proposed to approve this pCR for inclusion in TR 33.847.

# 2	References

[1] S3-213308 TR 33.847 solution #10 evaluation

# 3	Rationale

The contribution proposes to remove the following EN and adds text to the evaluation:

Editor's Note: Further evaluation is FFS.

It was proposed during SA3#104e to keep that EN because of the dependency on solution #10 and completion of its evaluation, which is proposed in S3-213308 [1] and available at this meeting.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.847.

***	BEGIN OF CHANGES	***

### 6.25.3	Evaluation

This solution proposes a mechanism to authorize a Remote UE to access a DN that requires a secondary A&A via L3 UE-to-Network relay using a network-controlled authorization procedure. This solution fully addresses the requirements in KI#4.

Impacts on 5GC and Remote UE/Relay UE to support the network-controlled authorization procedure are documented in sol#10 evaluation. Other KIs covered in sol#10 (e.g., KI#3, KI#5 etc) are not in scope of this solution.

For impact of secondary authentication support, the following capabilities are required:

- Remote UE has to support EAP authentication procedure via Relay UE

- Relay UE has to support relay of EAP authentication procedure towards the Remote UE.

- SMF has to be able to trigger Remote UE secondary authentication based on Remote UE subscription and perform secondary authentication for the Remote UE via Relay UE

The solution supports both Home Routed and Local breakout scenarios while reusing existing SMF selection mechanisms.

The solution supports Remote UE operating in or out of coverage.

The solution supports the roaming scenario while complying with LI requirements, i.e., allowing LI to take place in VPLMN using Remote UE's SUPI.

Editor's Note: the impact on 5GC and the existing procedures needs to be evaluated in consultation with SA2.

Editor's Note: Further evaluation is FFS.

***	END OF CHANGES	***