3GPP TSG-SA3 Meeting #104-e	S3-212721

# e-meeting, 16 – 27 August 2021

# Source:	Huawei, HiSilicon

# Title:	Update to the solution 12

# Document for:	Approval

Agenda Item:	5.11

1	Decision/action requested

This pCR proposes update to the solution 12 in TR 33.850.

2	Rationale

The evaluation is not complete. The explanation to the rekeying is added.

3	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.850.

********************** First Change ****************************

6.12	Solution #12: Protection of MBS traffic at service layer based on GBA

6.12.1	Solution overview

This solution addresses Key Issue 2 and 3 to protect the MBS key and traffic at service-layer. This solution leverages the MBMS security architecture specified in TS 33.246 [3].

6.12.2	Solution details

In order to receive an MBS service, the UE establishes a secure connection with the MBS service function and obtains security materials

Figure 6.12.2-1: Message flows for MBS key delivery and MBS traffic protection

0. The UE is registered to 5GS.

1. The UE requests a PDU session establishment or modification to receive an MBS service.

2. The UE establishes a secure connection with MBSF-U based on GBA similar to MBMS [3] or AKMA [5]. In both scenarios, MBSF-U is considered an AF and UE and MBSF-U communicate using Ua/Ua* protocol. Both the UE and MBSF-U derive Multicast User Key (MUK) from the AF key (e.g., Ks_(int/ext)_NAF for GBA or KAF for AKMA).

Editor’s Note: the details of authentication protocol between UE and MBSF-U is ffs.

Editor’s Note: It is FFS whether the key management function needs to be separated from MBSF-U or not.

3. The UE receives the Multicast Service Key (MSK) from the MBSF-U. The MSK is protected using the MUK and delivered using a unicast message over the secure connection.

4. The UE receives the Multicast Traffic Key (MTK) protected using MSK from the MBSF-U. The MTK can be delivered either a unicast or a multicast message. The MTK is used as a root key to derive application/protocol specific keys to protect (e.g., encrypt or integrity protect) MBS service traffic.

5. Using the MTK received in step 4, the UE derives application/protocol specific keys and decrypts or verifies the MBS traffic.

Editor’s Note: Use of AKMA for MUK derivation and rekeying requires further explanation.

The key hierarchy, rekeying and key usage for MBS traffic protection is illustrated in Figure 6.12.2-2 and Figure 6.12.2-3. The MUK derived either based on GBA or AKMA is used to protect MSKs, and each MSK is used to protect MTKs. MBSF-U decides to trigger the MSK/MTK update procedure based on the changes of authorization info or the key lifetime.  MSK rekeying is done over unicast to each UE joined to the MBS PDU session, and MTK rekeying is done over unicast or multicast to UEs joined to the MBS session.

Figure 6.12.2-2 Usage of MSK for a single session or a channel

Figure 6.12.2-3 Usage of MSK for multiple sessions or channels

6.12.3	Solution evaluation

This solution addresses Key issue #2 and 3 to protect the MBS key and traffic at service-layer.

MBSF-U decides to trigger the MTK update procedure based on the changes of authorization info or the key lifetime of MTK. The new MTK and KID are sent to UE and MBSF-U respectively. The MSK update procedure by MBSF-U to the UEs incurs overhead proportional to the number of UEs in connected state that joined the MBS session. The user plane messages need to be delivered to those UEs when a MTK change happens.

********************** End of pCR********************