3GPP TSG-SA3 Meeting #103-e 	S3-212855

# e-meeting, 16 - 27 August 2021														Revision of S3-21xxxx

# Source:	Qualcomm Incorporated

# Title:	Evaluation of Solution #24

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes to evaluate the Solution #24.

# 2	References

[1]	TR 33.847 v0.6.0

# 3	Rationale

This contribution proposes to evaluate the solution #24.

The NSSAA procedure is performed during the registration procedure of the UE, yet the remote UE in the L3 U2N relay case is not required to register to the relay UE’s serving network. Thus, this solution requires a new relayed NSSAA procedure that needs to be supported by the AMF and the Relay UE. Furthermore, the AMF should manage the context of remote UE and relay UE separately as their authorization state for the network slice(s) may be different (and independent).

Furthermore, it is unclear whether there are service requirements for NSSAA in SA2. Therefore, the following EN is added.

Editor’s Note: SA2 should be liaised to check whether there is a service requirement to support NSSAA of the remote UE by the relay UE’s serving network, in L3 U2N relay scenario.

Finally, further analysis on the impacts of the new relayed NSSAA on the existing network functions is required. Therefore, the following EN is added.

Editor’s Note: Impacts on the network functions are FFS.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

### 6.24.3	Evaluation

The NSSAA procedure is performed during the registration procedure of the UE, yet the remote UE in the L3 U2N relay case is not required to register to the relay UE’s serving network. Thus, this solution requires a new relayed NSSAA procedure that needs to be supported by the AMF and the Relay UE. Furthermore, the AMF should manage the context of remote UE and relay UE separately as their authorization state for the network slice(s) may be different (and independent).

Editor’s Note: SA2 should be liaised to check whether there is a service requirement to support NSSAA of the remote UE by the relay UE’s serving network, in L3 U2N relay scenario.

Editor’s Note: Impacts on the network functions are FFS.

Editor’s Note: further evaluation is FFS.

***** END OF CHANGES *****