3GPP TSG-SA3 Meeting #104-e 	S3-213215

e-meeting, 16 - 27 August 2021

Title:	LS on SBA for GBA

Response to:

Release:	Release 17

Work Item:	GBA_5G

Source:	SA3

To:	SA2

Cc:

Contact person:	Vlasios Tsiatsis

vlasios dot tsiatsis at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3-213245 (Living document of TS 33.220), S3-213246 (Living document of TS 33.223).

# 1	Overall description

SA3 has decided to add new clauses in TS 33.220 and TS 33.223 for the architecture and interfaces as well as the reference point interfaces for the specification of the SBA version of the GBA and GPA Push. As part of the architecture specification, reference point architecture and interfaces were introduced in the attached draft CRs S3-213245 and S3-213246. In addition, new clauses were introduced for the discovery and selection of HSS, BSF and UDM in the aforementioned draft CRs.

SA3 has the following two requests.

Request 1) Reference point interface and SBI names: The SBA architecture for GBA includes the following reference point interfaces and SBA interfaces with the following proposed interface names:

NG1:Reference point between an SBI capable BSF and an SBI capable HSS.

NG2: Reference point between an SBI capable BSF and an SBI capable NAF.

NG3: Reference point between an SBI capable BSF and an SBI capable Push-NAF, i.e. a Push-NAF that supports an SBI interface towards an SBI capable BSF.

NG1': Reference point between an SBI capable BSF and a UDM.

Nbsp: Service-based interface exhibited by an SBI capable BSF.

It must be observed that the BSF in this context stands for the Bootstrapping Server Function and it is related to GBA (TS 33.220) and GBA Push (TS 22.223) which was defined already in 3G/4G. Since SA2 defined a network function called "BSF" (Binding Support Function in TS 23.501) SA3 considers that there might be an issue with the SBA interface names of the GBA BSF as opposed to the ones for the Binding Support Function. Therefore, SA3 has decided to name the SBI for the GBA BSF as "Nbsp".

SA3 kindly requests SA2 to take the information into account about the names of the reference point interfaces and the Service Based Interfaces.

Request 2) SA3 is considering adding selection clauses for the HSS, BSF and UDM, in both the draft CRs to TS 33.220 and TS 33.223.

It must be observed that SA3 considers a new discovery and selection clause for the SBI capable HSS compared to the corresponding clause in Annex AA, TS 23.228 (IMS) since the selection criteria are different in the two clauses.

SA3 kindly requests SA2 to review the discovery and selection clauses of HSS, BSF and UDM and provide feedback if there are issues.

# 2	Actions

To SA2

ACTION: 	SA3 kindly requests SA2 to take the information into account about the names of the reference point interfaces and the Service Based Interfaces.

ACTION: 	SA3 kindly requests SA2 to review the discovery and selection clauses of HSS, BSF and UDM and provide feedback if there are issues.

# 3	Dates of next TSG SA WG 3 meetings

SA3#104-e Ad-hoc	September 27-30, 2021		Electronic meeting

SA3#105-e		November 8-19, 2021		Electronic meeting