3GPP TSG-SA3 Meeting #104-e 	draft_S3-212547-r1

- e-meeting, 16 - 27 August 2021											Revision of S3-20xxxx

- Source:	ZTE Corporation

- Title:	Update to Solution #5

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This document proposes to update Solution #5 to the TR 33.847. SA3 is kindly requested to approve this document.

# 2	References

[1]	3GPP TR 33.847, v 0.6.0.

# 3	Rationale

Besides TLS, other Ua* protocol profiles may be defined for AKMA. This contribution proposes to update Solution #1 to the TR 33.847 to allow other Ua* protocol profiles to be identified to protect the PC3 interace along whith AKMA.

The EN in the Evaluation clause is deleted.

A typo is corrected.

# 4	Detailed proposal

***	BEGIN OF CHANGE	***

## 6.5	Solution #5: Protection of the PC3 interface using AKMA and TLS

### 6.5.1	Introduction

This solution describes how AF in AKMA TS 33.535 [7] can be used to generate the key to be used to protect the PC3 interface between the UE and the 5GDNNMF. This solution addresses key issue #10.

This solution can also be used with other AF’s used for ProSe services which are accessed in the user plane.

### 6.5.2	Solution details

This solution assumes that 5GDDNMF is a separate entity and not a functionality of the PCF. This solution assumes that the 5GDDNMF takes the role as the AF in AKMA and uses AKMA procedures as defined in TS 33.535 [7] to generate a symmetric key (i.e. KAF) in the UE and the AF.

This solution proposes to use TLS 1.3 with PSK authentication as described in RFC 8446 [11].

Editor's Note: It is FFS whether other Ua* protocol profiles may be identified to protect the PC3 interface along with AKMA.

The signaling flow in clause 6.5.2-1 describes the establishment of TLS 1.3 with PSK authentication.

There is no separate authentication of the UE to support AKMA functionality. Instead, it reuses the 5G primary authentication procedure executed e.g. during the UE Registration to authenticate the UE. A successful 5G primary authentication results in KAUSF being stored at the AUSF and the UE.

The AUSF generates KAKMA from KAUSF and generates an A-KID which is mapped to the new generated KAKMA and pushes the KAKMA and A-KID to the AAnF.

Figure 6.5.2-1: Procedure for security protection of the PC3 interface between the AF (e.g. 5GDDNMF) and the UE

Step 1) The UE mutually authenticates with and registers in the 5GC. As part of the UE authentication with the 5GC, the UE and the AUSF store a KAUSF. Additionally, the UE and the AUSF generate AKMA Key material (i.e. KAKMA and A-KID) and the AUSF sends this material to the AAnF as specified in the AKMA TS 33.535 [7]. The UDM will indicate whether the UE is allowed to use AKMA services. The PCF (or some other network function) provides the AF address (e.g 5GDNNMF address) to the UE and the UE establish PDU session with the network.

Step 2a) The UE initiates TLS 1.3 with PSK authentication with the AF server as described in RFC 8446 [11. The UE sends Client Hello where the ClientHello contains a pre_shared_key extension containg a PSK identity formatted from A-KID and 3GPP-akma hint together with a psk_key_exchange_modes extension indicating e.g. psk_dhe_ke.

The following steps in 2 b)-c) are part of AKMA procedures defined in TS 33.535 [7].

Step 2b) The AF server contacts the AAnF with the A-KID.

Step 2c) The AAnF looks up the KAKMA key using the A-KID and generates a KAF key from the KAKMA key.

Step 2d) The AAnF server responds with the KAF key and the expiration time for the KAF key to the AF.

Step 2e) The AF server responds with a Server Hello with a pre_shared_key extension indicating the chosen PSK identity.

Step 2f) The UE generates KAF from KAKMA.

Step 2g) The UE responds with a Finished message.

Step 3 The UE and the AF server can exchange application data over a secured link.

### 6.5.3	Evaluation

SA2 has adopted the user plane architecture described in TR 23.752 [2].

This solution resolves key issue #10 and can secure PC3 interface using AKMA which is specified in TS 33.535 [7] in Rel-17.

***	END OF CHANGE	***