3GPP TSG-SA3 Meeting #104-e 	draft_S3-202812-r1

- e-meeting, 16 – 27 August 2021					                                      Revision of S3-20xxxx

- Source:	China Mobile

- Title:	Solution to abnormal behaviour detection of IoT SIM card

- Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

This contribution proposes a solution to abnormal behaviour detection of IoT sim card

# 2	References

[1]	 3GPP TS 23.288 Architecture enhancements for 5G System (5GS) to support network data analytics services

# 3	Rationale

This contribution proposes a solution to abnormal behaviour detection of IOT SIM card

# 4	Detailed proposal

*************** Start of 1st Change ****************

- 6.X	Solution #X Solution to abnormal behaviour detection of IoT SIM card

- 6.X.1	Introduction

This solution aims at addressing Key Issue #2.1 Cyber-attacks detection supported by NWDAF.

The continuous growth of the IoT business has brought more problems to the management and safety supervision of the IoT USIMs. Especially in the case of 5G networks, the number of mIoT users has increased sharply, attackers may use IoT USIMs for abusive operations where the IoT USIMs are used with unauthorized MEs, but the existing 5G network lack a design process for identifying those IoT USIMs used with unauthorized MEs.

- 6.X.2	Solution details

This solution analyzes abnormal changes in the location of the UE containing IoT USIM in a specific industry.The IoT USIMs applied industry can be indentified by the Offer_ID data of the IoT card. Since the positon of IoT USIMs for certain industries should be fixed, when position information (such as TAI and Cell_ID) of IoT USIMs for some industry users (such as electric cards, water cards, etc.) have changed multiple times within a period of time, it indicates there is an abnormal location update for the IoT USIMs.

- An external application (which can be a CRM) send Offer_ID,GPSI and IoT_Behavior of the IoT card to NEF through Nnef_AnalyticsExposure_subscribe.

- NEF receive  " IoT_Behavior " and then set the Analytics ID to "Abnormal behaviour", set the exception ID to "Abuse of IoT SIM card", and then request analytic to NWDAF.

  - NWDAF request and get the correpongding SUPI from UDM

5-6. NWDAF request and get the TAI and CELL_ID of the IoT card from AMF.

7. NWDAF produces analytics.

8-9. NWDAF send the analytic result and the GPSI back to AF via NEF

- Table 6.X: Description of Expected UE Behaviour parameters per Exception ID

| Exception ID | UE behaviour parameters to provide | NF for data collection |
| --- | --- | --- |
| Abuse of IoT SIM card | TAI CELL_ID | AMF |
|  | SUPI | UDM |

Table 6.X Inputs provided to NWDAF in assisting the detection of IOT sim card abnormal behaviour

| Information | Source | Description |
| --- | --- | --- |
| SUPI | UDM | The corresponding SUPI of the IoT SIM card refer to the GPSI |
| TAI | AMF | TAI of the IoT SIM card to detect the location |
| CELL_ID | AMF | CELL_ID of the IoT SIM card to detect the location |

- 6.X.3	Evaluation

NOTE: USAT pairing mechanism is specified in 3GPP TS 33.187 to restrict the USIM to specific UEs to address this kind of attack.

*************** End of 1st Change ****************