3GPP TSG-SA3 Meeting #104-e 	S3-212483

- e-meeting, 16 - 27 August 2021

- Source:	Johns Hopkins University APL, US National Security Agency, CableLabs, InterDigital, CISA ECD, Verizon

- Title:	New Solution to Network Slice Quota DoS

- Document for:	Approval

Agenda Item:	5.21

# 1	Decision/action requested

It is requested to approve this contribution for a solution in TR 33.874

# 2	References

[1]	3GPP TR 23.502: “Procedures for the 5G System (5GS); Stage 2 (Release 17)”

# 3	Rationale

This contribution proposes a solution to the DoS vulnerability in the Network Slice Quota functionality, as defined in TS 23.502 [1] clause 4.2.11 Network Slice Admission Control Function (NSACF) Procedures.

# 4	Detailed proposal

Note to rapporteur: all text in the change below is new.

****Start of Change****

## 6.Y	Solution #Y: Solution to Network Slice Quota DoS

### 6.Y.1	Introduction

This solution addresses DoS attacks against the network slice quota managed by the NSACF. At the core of the vulnerability is a lack of validation of the message content sent to the NSACF during the procedure used to update the number of UEs registered with a S-NSSAI. The two message fields that need to be validated are the UE ID and the S-NSSAI. The NSACF needs to validate that the purported UE ID is valid for the 5GS. The NSACF also needs to validate that the purported S-NSSAI is allowed for the UE ID.

### 6.Y.2	Solution Details

The NSACF will have an access control list, separate from the lists used to maintain slice quotas, that keeps track of which UE IDs are permitted on each S-NSSAI that is using the UE quota functionality. Upon receiving the Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Request message, the NSACF queries the access control list to validate that the UE ID is valid for the 5GS and that the UE ID is permitted on the S-NSSAI.

### 6.Y.3	Evaluation

Editor’s Note: Evaluation is FFS.

****End of Change****