3GPP TSG-SA3 Meeting #104-e 	draft_S3-212556-r1

- e-meeting, 16 - 27 August 2021											Revision of S3-20xxxx

- Source:	ZTE

- Title:	Update the solution #6

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

This contribution proposes to update the solution #6.

# 2	References

[1]	3GPP TR 33.850 "Study on Security Aspects of Enhancements for 5G Multicast-Broadcast Services (MBS)".

# 3	Rationale

This contribution proposes to update the solution #6. This contribution try to resolve the ENs in the solution.

# 4	Detailed proposal

**** 1st CHANGE ****

## 6.6	Solution #6: Authentication and authorization for multicast communication service based on AKMA

### 6.6.1	Solution overview

This solution, which is based on AKMA, addresses the key issue #1 Security of authentication and authorization for multicast communication service.

### 6.2	Solution details

- Figure 6.6.2-1 Authentication between the MBSF/MSF-C and UE based AKMA

- UE shall generate the AKMA Anchor Key (KAKMA) and the A-KID from the KAUSF before initiating communication with an AKMA Application Function, i.e MBSF/MSF-C, as specified in TS 33.535 [5].

- UE derive a key KMBS for authentication with the MBSF/MSF-C .

- When UE try to join the multicast service, UE computes MAC-I and then UE sends a MBS service request to MBSF/MSF-C. The service request include A-KID and MAC-I.

- Editor’s Note: How to derive the MAC-I is FFS.

- 3-6. Upon receiving the request, the MBSF/MSF-U discovers the AAnF, then AAnF generates KMBS and sends the KMBS to MBSF/MSF-C. The AAnF discovery and selection is specified in the TS 33.535[5] clause 6.7.  The MBSF/ MSF-C computes the MAC-I using KMBS, and check if it is the same with the received MAC-I from UE. If so,  it means that the UE is authenticated and authorized to use the service.

Editor’s Note: How MBSF/MSF-C obtains authorization information is FFS.

- 7.  The MBSF/MSF-C verifies the MAC-I using the KMBS, when the verification is succeed, and if the UE is authorized to perform the operation,Then the MBSF/MSF-C sends a service response to the UE.

### 6.6.3	Solution evaluation

TBD

**** END OF CHANGES ****