- 3GPP TSG-SA3 Meeting #104e	draft_S3-212614-r1

- e-meeting, 16 – 27 Aug 2021	revision of S3-21xabc, Merger of S3-212469

- Source:	Huawei, HiSilicon

- Title:	Add conclusion to Key Issue #12

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

Approve the pCR below to add conclusion to Key Issue #12 in TR 33.847

# 2		Rationale

This contribution aims to conclude Key Issue #12. This contribution reflects the agreed potential baseline during the teleconference, that is:

‘Rapporteur: V2X security defined in TS 33.536 shall be the baseline for normative work, plus some enhancements introduced in Sol#7, Sol#23, Sol#27, Sol#28, and Sol#33 if they are approved by SA3.

Decisions: As above, need to solve the bidding down attack.’

# 3	Detailed proposal

#### *** BEGIN CHANGES ***

## 7.X	Conclusions for Key Issue #12

For Key Issue #12, the following is taken as conclusions:

-	If the principle for the eV2X PC5 unicast link establishment in TS 33.536 [8] is reused (i.e. both signalling and user plane have on-demand security negotiation), it is concluded that the bidding-down attack caused by unprotected PC5 signalling link needs to be solved in the normative phase.

-	It is concluded that the security mechanism in Solution #7 is taken as the basis for normative work if signalling has mandatory security protection and user plane uses on-demand security.

-	It is concluded that the one-to-one communication establishment failure caused by security policy mismatch needs to be solved in normative phase by using Solution #27 and Solution #28 as the basis or developing new solutions.

-	The following security mechanisms in Solution #23 is taken as basis for normative work to provide means for secure refresh of UE security context: During the direct discovery with 5G DDNMF for direct communication, UE gets the keys for PC5 connection and the associated key validate time.

NOTE: Further details for secure refresh of UE security context will be finished during the normative phase.

#### *** END OF CHANGES ***