- 3GPP TSG-SA3 Meeting #103-e 	draft_S3-212644-r8

- e-meeting, 16 – 27 August 2021, Online

- Source:	Huawei, HiSilicon, Ericsson?, Qualcomm?, Samsung? Docomo? Apple? Thales? ZTE? Lenovo?...

- Title:	EC: Conclusion for Key issue #2

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes the conclusion for KI#2.

# 2	References

[1] TR 33.839 v040

# 3	Rationale

5.2.3	Potential Security Requirements

Edge Configuration Server(ECS) Requirements:

Edge Configuration Server(ECS) shall be able to provide mutual authentication with Edge Enabler Client over EDGE-4 Interface.

Edge Configuration Server(ECS) shall be able to determine whether Edge Enabling the client is authorized to access provisioning services offered by Edge Configuration Server(ECS).

It is proposed to conclude the solution for this requirement.

# 4	Detailed proposal

It is suggested to approve the following changes.

***************************Start of change ****************************

### 7.2	Conclusions for Key Issue #2

It is concluded are as follows:

For the Authentication issue between EEC and ECS, it is suggested to use two alternative mechansims for the normative work: the Server (ECS) side certificate authentication using TLS + Client (EEC) side authentication using application layer based mechanism (such as token) generated by the ECSP or a third party described in solution #17.

NOTE 1: Using TLS as described above also allows certificate-based client authentication to be performed if the UE has been provisioned with suitable keys and certificate(s). No specific method of provisioning the keys and certificate(s) is to be standardised.

Editor’s Note: Whether AKMA and GBA are applicable here is FFS.

For the Authorization issue between EEC and ECS, it is suggested to use the static authorization by the ECS for the normative work.

Editor’s Note: Conclusion for the GPSI verification issue is FFS.

***************************End of change****************************