- 3GPP TSG-SA3 Meeting #104-e 	S3-212649-r1

- e-meeting, 16 – 27 August 2021, Online

- Source:	Huawei, HiSilicon

- Title:	TS 33.558: Security for the EDGE interfaces

- Document for:	Approval

Agenda Item:	4.18

# 1	Decision/action requested

This contribution proposes to add the security for the EDGE interfaces.

# 2	References

[1] TR 33.558

# 3	Rationale

It is proposed to add the Security for the EDGE interfaces according to the conclusion for key #6 in TR 33.839.

# 4	Detailed proposal

It is suggested to approve the following changes.

*************** Start of the 1st change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[xx]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[yy]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[zz]	3GPP TS 33.187: "Security aspects of Machine-Type Communications (MTC) and other mobile data applications communications enhancements".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

*************** Next change ****************

## 6.1	Security for the EDGE interfaces

For the interfaces (EDGE-1/4), TLS specified in TS 33.210 [xx] shall be used if HTTP protocol is selected.

For the interfaces EDGE-2/7/8,

- if the NEF APIs is selected, security aspects of Network Exposure Function including the protection of NEF-AF interface and support of CAPIF defined in TS 33.501 clause 12 [yy] shall be reused, i.e. use of TLS.

- if the SCEF APIs is selected, the Security procedures for reference point SCEF-SCS/AS defined in TS 33.187 clause 5.5 [zz] can be reused here, i.e. use of TLS.

For the interfaces (EDGE-3/6/9), TLS shall be used as specified in TS 33.210 [xx], unless security is provided by other means, e.g. physical security.

*************** End of the 1st change ****************