- 3GPP TSG-SA3 Meeting #104-e 	draft_S3-212651-r3

- e-meeting, 16 – 27 August 2021, Online

- Source:	Huawei, HiSilicon, China Mobile

- Title:	New Key issue on authorization mechanism negotiation

- Document for:	Approval

Agenda Item:	5.20

# 1	Decision/action requested

This contribution proposes to add a new key issue on authorization mechanism negotiation.

# 2	References

[1] TR 33.875 v020 Study on enhanced security aspects of the 5G Service Based Architecture (SBA)

# 3	Rationale

It is specified in TS 33.501 [2] clause 13.3.0 that static authorization can be used for authorization when token-based authorization is not used. However, two PLMNs may have the roaming issue if the authorization mechanism is not aligned between them. For example, when the NF service consumer (NFc) deployed in one PLMN only supports static authorization, and the NF service producer (NFp) deployed in the other PLMN only supports OAuth authorization, the NFp will reject the NF service consumer.

On the other hand, TS 29.510 [xx] defined an oauth2Requried indicating that OAuth authorization is required for the NFp service access, which will be sent back to the NFc via the discovery response. Accordingly, NFc shall get the token before consuming the NFp services. Hence, NFc that only supports the static authorization will not be able to consumer the service provided by the NFp. However, how to handle the failure issue when the NFc only support static authorization is not clarified

# 4	Detailed proposal

It is suggested to approve the following changes.

*************** Start of the 1st change ****************

## 5.x	Key issue #x: Authorization mechanism handling in roaming

### 5.x.1	Key issue details

It is specified in TS 33.501 [2] clause 13.3.0 that static authorization can be used for authorization when token-based authorization is not used. However, two PLMNs may have the roaming issue if the authorization mechanism is not aligned between them. For example, when the NF service consumer (NFc) deployed in one PLMN uses the static authorization, and the NF service producer (NFp) deployed in the other PLMN uses the OAuth authorization, the NFp will reject the NF service consumer.

On the other hand, TS 29.510 [xx] defined an oauth2Requried indicating that OAuth authorization is required for the NFp service access, which will be sent back to the NFc via the discovery response. Accordingly, NFc shall get the token before consuming the NFp services. Hence, NFc that uses the static authorization will not be able to consume the service provided by the NFp. However, how to handle the failure issue when the NFc uses the static authorization is not clarified.

The key issue will investigate solutions allowing the two operators to handle the case that one operator uses token-based authorization and its roaming partner uses static authorization.

### 5.x.2	Security threats

The SBA service authorization will fail in the roaming case if the authorization mechanism is not aligned between them.

### 5.x.3	Potential security requirements

The 5GS should provide mechanisms to handle the case that one operator uses token-based authorization and its roaming partner uses static authorization.

*************** End of the 1st change ****************