3GPP TSG-SA3 Meeting #104-e 	S3-212744

- e-meeting, 16 – 27 August 2021

- Source:	Philips International B.V.

- Title:	Clarification quantum resistance

- Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

Acceptance of additions regarding quantum resistance. This text could be added in Solution #20 or in an appendix of TR33.809.

# 2   References

[x] NISTIR 8309: “Status Report on the Second Round of the NIST Post-Quantum Cryptography Standardization Proces”. July 2020.  Available online at https://doi.org/10.6028/NIST.IR.8309

[y] Neish, Andrew, Walter, Todd, Enge, Per, "Quantum Resistant Authentication Algorithms for Satellite-Based Augmentation Systems," Proceedings of the 2018 International Technical Meeting of The Institute of Navigation, Reston, Virginia, January 2018, pp. 365-379

# 3   Rationale

This proposal includes a brief assessment related to quantum resistance.

- The second change is generic, and thus, it is suggested to add it to Appendix A. This text can include other references to previous work on quantum resistance or include in the future new results.

- The first change is more specific to Solution #20, and thus, it can be included in Section 6.20.3.

# 4	Detailed proposal

The changes can be made in two alternative places (1st change or 2nd change) to be discussed during the meeting.

***	BEGIN OF 1st CHANGE	***

### 6.20.3	Assessment using clause A.3

#### 6.20.3.1	UE aspects

UE needs to be provisioned of a list of trust anchors (see clause 6.20.2.5.1).

UE should support secure storage of trusted anchors.

UE needs to take into consideration results from signature and freshness verification of SI for cell selection and reselection (see clause 6.20.2.5).

Editor Note: further assessment is required, e.g., SIB acquisition, (gNB) certificate/key acquisition depending on the solution variants.

#### 6.20.3.2	UE actions upon detection of invalid signature

Upon detection of invalid signature, UE should not select the cell as described in clause 6.20.2.5.5.

#### 6.20.3.3	Threats that are mitigated by signed SI messages

Man-on-the-side attacks (e.g., SigOver), man-in-the-middle attacks (e.g., replay and relay), and some denial of services (e.g., from tampering with SI) are mitigated.

#### 6.20.3.4	Threats that are not mitigated by signed SI messages

Some denial of services (such as from bitflipping or radio jamming of all available cells) cannot be mitigated.

#### 6.20.3.5	Provisioning of keys

Trusted anchors need to be provisioned into UE during manufacture, onboarding, or after registration.

#### 6.20.3.6	RAN aspects

gNB needs to request digital signatures and obtain short-lived keys from DSnF.

gNB needs to sign dynamic fields (e.g., SFN) using short-lived keys.

gNB needs to broadcast digital signatures along with SIs

Editor Note: further assessment is required, e.g., certificate/key broadcast depending on the solution variants.

#### 6.20.3.7	VPLMN aspects

If the trust anchor of VPLMN is provisioned into a UE, the UE is protected when accessing the VPLMN.

#### 6.20.3.8	HPLMN aspects

If the trust anchor of HPLMN is provisioned into a UE, the UE is protected when accessing the HPLMN.

#### 6.20.3.9	Network sharing aspects

When a gNB is shared by multiple PLMNs, the operator owning the gNB can sign SIs as long as the trust anchor of the gNB operator is provisioned into a UE.

#### 6.20.3.10	Roaming aspects

See clause 6.20.3.6 VPLMN aspects.

#### 6.20.3.11	Regulatory aspects

Regulatory requirements, if there are any, can be supported.

#### 6.20.3.12	Signature schemes

Potential signature schemes include:

-	ECDSA (recommended with named curves)

Editor's Note: the ECDSA profile for SUCI can be reused.

-	RSA

-	others

NOTE: The hash based approach in 6.20.2.2.4 only requires a hash function.

#### 6.20.3.13	Signature length

RSA: 256 bytes

ECDSA: 64 bytes

#### 6.20.3.14	Resistance against Quantum Computing

Next to the general remarks in Appendix A.3.14 and Table A.3.14-1, hash functions are only mildly affected by quantum computers [x]. Thus, the hash based approach in 6.20.2.2.4 is a potential quantum resistant solution for suitable parameters. See [y] for a related application. Note that this requires the trust anchor of the hash chain to be securely deployed by means of a quantum resistant algorithm.

The expected performance and properties of quantum resistance algorithms and solutions should be considered to facilitate interoperability in the future. For instance, Table 6.20.3.14-2 describes the features (pros/cons) of different potential configurations based on Section 6.20.2.2.4 assuming some specific digital signing algorithms.

Table 6.20.3.14-2

| Configuration | DNsF signing algorithm | gNB signing algorithm | Features |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Quantum resistant | Message size to distribute the short-term certificate | Do short-term certificate and/or signature fit in a single SIB? |
| #1 | ECDSA | ECDSA | No | Short: ECDSA signature using DSnF’s ECDSA private key on a short-term gNB ECDSA public-key | Yes |
| #2 | ECDSA | Hash Chain-based | No | Short: ECDSA signature using DSnF’s ECDSA private key on the new gNB’s hash chain anchor. | Yes |
| #3 | Rainbow | Rainbow | Yes | Long: Rainbow signature using DSnF’s rainbow private key on a short-term gNB’s rainbow public-key (long) | No: 1) the signature fits; 2) the short-term certificate does not fit in a single SIB. |
| #4 | Rainbow | Hash Chain-based | Yes | Short: Rainbow signature using DSnF’s Rainbow private key on the new gNB’s hash chain anchor. | Yes |
| #5 | Falcon | Falcon | Yes | Medium: Falcon signature using DSnF’s Falcon private key on a short-term gNB Falcon public-key | No: 1) a signature generated by the gNB does not fit in a SIB; 2) the short-term certificate does not fit in a single SIB. |
| #6 | Falcon | Hash Chain-based | Yes | Medium-short: Falcon signature using DSnF’s Falcon private key on the new gNB’s hash chain anchor. | No: 1) only the signature fits; 2) the short-term certificate does not fit in a single SIB. |

Based on the numbers in Table A.3.14-1, the expected sizes for the public key + signature of configuration #4 (short), configuration #3 (medium), and configuration #5 (long) are 114 bytes, 1563 bytes, and 58.9 kB, respectively. The first value assumes that the hash chain anchor is 48 bytes long.

***	END OF 1st CHANGE	***

***	BEGIN OF 2nd CHANGE	***

## A.3.14	Resistance against Quantum Computing

Quantum computers will severely affect digital signing algorithm based on factoring, discrete logarithms, and elliptic curve cryptography [x] such as RSA or ECDSA. Therefore, resistance against quantum computing requires using quantum resistant solutions. Quantum resistant signature algorithms have bigger public keys and signatures than ECDSA or RSA as reflected in Table 6.20.3.14-1, which includes the key sizes of three of the existing signature algorithms with their smallest security parameters. These bigger key/signature sizes might affect the choice/performance of the solution once the solution (or other digital signature solutions) is required to start using quantum-resistant signature algorithms.

Table A.3.14-1

| Algorithm | Public key size | Signature size | Source |
| --- | --- | --- | --- |
| Dilithium | 1312 bytes | 2420 bytes | https://pq-crystals.org/dilithium/index.shtml |
| Falcon | 897 bytes | 666 bytes | https://falcon-sign.info/ |
| Rainbow | 58.8 kB | 66 bytes | https://www.pqcrainbow.org/ |

***	END OF 2nd CHANGE	***