3GPP TSG-SA3 Meeting #104-e 	S3-212868-r1

- e-meeting, 16 - 27 August 2021											Revision of S3-20xxxx

- Source:	Qualcomm Incorporated

- Title:	pCR: Additional conclusions for KI #1

- Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

Approve this contribution to further conclude on Key issue #1 in TR33.857

# 2	References

[1]	TR 33.857

# 3	Rationale

For Key issue #1 (Credentials owned by an external entity), when the EAP authentication is used with a legacy AAA server, the UE needs know that KAUSF needs to be derived from MSK instead of EMSK. It is proposed to conclude that the UE is provisioned with an indication that the KAUSF key derivation needs to be performed from MSK instead of EMSK.

This approach has no impact on any of the existing network entities or network interfaces.

Note that this indication is needed only if legacy AAA server is used. If legacy AAA server is not used, no indication needs to be configured. The exact provisioning process or method can be left to the implementation and need not be specified by 3GPP.

# 4	Detailed proposal

SA3 is kindly requested to approve the below pCR to [1].

***	BEGINNING OF CHANGES ***

## 7.1	Conclusions on KI #1: Credentials owned by an external entity

In case that the external entity is 5GS aware (i.e., has the AUSF/UDM and is capable of deriving 5G key hierarchy after a successful primary authentication), it is concluded that the existing 5GS roaming architecture is reused.

In case that the external entity is non-5GS aware (legacy AAA server), the following is concluded:

- The SNPN access with a credential owned by an external entity is performed via an AUSF in the SNPN that is enhanced to interface with the external entity.

- The UE is provisioned with an indication that the key KAUSF needs to be derived from MSK instead of EMSK when authenticating using the credential owned by the external entity. The provisioning of this indication is done together with the provisioning of the credential owned by the external entity.

Editor’s Note: Further conclusion(s) are FFS.

***	END OF CHANGES ***